DEA LOGO

SOUTH AFRICA’S

NATIONAL OIL SPILL CONTINGENCY PLAN

(NOSCP)

Prepared by:

Department of Transport

Department of Environmental Affairs

South African Maritime Safety Authority

...................................                              .........................................

Mr J.S.  Ndbele			   		Ms. E. Molewa

Minister of Transport				Minister of Environmental Affairs

Date:						Date:

| Date | ??/08/2011 |
| --- | --- |
| Amendment | 1 |

| Date | ??/08/2011 |
| --- | --- |
| Amendment | 1 |

AMENDMENT RECORD

| No: | Date of issue: | Pages Amended: |
| --- | --- | --- |

AMENDMENTS

All amendments to this document will be issued by the (title of person in SAMSA)  in the form of new pages which are to replace the amended pages.  The holder of each copy of this plan as named in the Distribution List will receive a copy of the new pages.  The amendments are highlighted by a vertical line in the left hand margin and amended pages will be dated with a foot note.

Upon receipt of the amendments, the recipient is to verify its completeness and update his copy of the Plan accordingly, and then mail, or fax, the tear-off slip to the addressee given at the top of the tear-off slip.  (See Appendix II)

Holders of the plan are to advise the (TITLE OF RESPONSIBLE PERSON AT SAMSA of any pertinent changes of telephone numbers, organisational structure or other relevant information as soon as it comes to their notice.

PREFACE

South Africa is situated at the interface of the southern Atlantic and Indian oceans and its 3 000km coastline is one of the oldest navigation routes of the world.  This route has a large volume of passing traffic that does not necessarily call at South African ports.  In addition, about 19 million tons of crude oil is imported into South Africa annually, while approximately  120  million  tons  passes  around  the  Cape  bound  for  world markets.  .

Historically the Cape of Good Hope was named “Cape of Storms” for good reason, and many ancient and contemporary vessels have foundered off the hazardous South African coastline.  One of the largest crude oil spills to have occurred anywhere in the world, took  place off Saldanha Bay in 1983 when the Spanish tanker “Castillo de Bellver” broke in half and sank with  224 000 tons of light crude on board.  More recent spills have involved the release of heavy bunker fuels from dry cargo vessels.

All marine traffic, calling at South African ports or in transit about the coast, presents a risk of marine pollution resulting from collisions, groundings, oil cargo and bunker transfers, structural failure or any other number of maritime emergencies or accidents.  Many cargoes carried by these vessels present an additional risk, as they may contain any number or combination of harmful substances and these marine pollutants generally threaten recreation areas, sea birds, marine life, coastal installations and fisheries.

Experience has shown that large oil spills are coupled with some form of a casualty such as a grounding, hull failure, collision, etc.  No two casualties (incidents) are the same and so response actions may vary considerably depending on the nature of the casualty, the type of oil that has been spilled, the location of the spill, hazardous cargo spilled, sea conditions, etc.  To date no major spill has occurred off the South African coast arising from an offshore installation.

The implementation of this Plan seeks to provide effective prevention of oil spills off the South African coastline  and the minimisation of the impact of any pollution from ships and offshore installations.

It is therefore the intention of this Plan to specify the broad principles of South Africa’s prevention and response policy and to  allow the designated response personnel, to deal with the incident as the circumstances dictate according to their own more specific response plans.

Distribution List

- Department of Transport (DoT)

- Department of Environmental Affairs (DEA)

- Department of Environmental affairs(DEA) - Oceans & Coasts

- Department of Agriculture Forestry & Fisheries(DAFF)

- South African Maritime Safety Authority (SAMSA)

- South African Navy (SAN)

- SANPARKS

- Transnet

- Transnet National Ports Authority

- Provincial Disaster Management

- PetroSA

- KZN : SAPREF

- P&I Club

- Oil pollution control South Africa (OPCSA)

- South African Petroleum Industry Association (SAPIA)

Please advise any omissions

INDEX

Page no.

PART I: SCOPE AND PURPOSE, DEFINITIONS AND LEGISLATION

1.1	Scope and purpose

1.2	 Geographic coverage of Plan

1.3	Definitions

1.4	Conventions, Acts and Agreements relating to the Prevention and Combating of Oil Pollution at Sea

1.4.1	International Conventions

1.4.1.1 Oil Pollution

1.4.1.2 Compensation for Oil Pollution Damage

1.4.1.3  Operational Requirements

1.4.2  National Legislation

PART II: POLICY AND PLANNING

2	Organisational Arrangements

2.1	National Organisation (SAMSA AND DEA)

2.2	Responsibilities of SAMSA and DEA

2.3	General application of the division between SAMSA and DEA

2.4	Finance and compensation

2.5	The Joint Response Committee (JRC)

2.6.	Support agencies and other stakeholders

2.6.1	Local Authorities

2.6.2	South African internal and territorial waters

2.6.3	Waters beyond the territorial seas

2.6.4	Ports

2.6.5	Terminals

2.6.6	Spills from Offshore Installations

2.6.7	Spills of any other harmful substances

2.6.8	Other stakeholders and role players

2.7	Geographical responsibilities

2.8	International assistance

PART III: PREVENTION STRATEGY

3	Pollution Prevention

3.1	Casualty management

3.2	Places of refuge

3.3	Reporting and alerting

3.4	SAMSA’s action plan

3.5	SAMSA casualty response unit

3.6	The Joint Response committee

3.7	Termination of the response

PART IV: COMBATING STRATEGY

4.1	Levels of response

4.2	Risk assessment

4.3	Protection priorities

4.4	Response structure

4.5	National combating strategies

4.5.1	Surveillance

4.5.2	Containment and recovery

4.5.3	Dispersion

4.5.4	Shoreline protection and clean-up

4.5.5	Bioremediation

4.5.6	Rocky shoreline clean-up

4.6	Oil combating equipment

4.7	Disposal of pollutants and debris

PART V: ANCILLARY ACTIVITIES

5	Media Response

5.1	Press conferences

5.2	Ministerial and VIP visits

6	Record keeping and management

7	Reporting

8	Training and exercises

9	Maintenance and review of the plan

Appendixes

Appendix 1	Local coastal contingency plans

Appendix 2	Telephone list for oil spill reporting

Appendix 3	Procedure for responding to chemical or HNS spills

Appendix 4	Procedure for setting up the JRC

Appendix 5	JRC management Protocol

Appendix 6	DEA procedure for the mobilisation of equipment

Appendix 7	Procedure for integrating “lessons learnt” into the plan

Appendix 8	SAMSA initial response check list

Appendix 9	DEA initial response check list

Appendix 10	Limitation of liability

Appendix 11	Initial Hazardous Noxious Substance Spill Report

# Appendix 12

Addendum	RSA Pollution Equipment

Other 2 Appendixes??

What about more flow charts??

PART I: SCOPE AND PURPOSE, DEFINITIONS AND LEGISLATION

1.1	Scope and Purpose

The purpose of the National Oil Spill Contingency Plan ( the Plan) is to outline the national policy /strategy for responding to oil pollution incidents affecting the marine environment.

The primary Act dealing with the prevention and combating of pollution in the South African scenario is the Marine Pollution (Control and Civil Liability) Act 6 of 1981.  In its preamble, this Act states that its purpose is: “To provide for the protection of the marine environment from pollution by oil and other marine harmful substances, and for that purpose to provide for the prevention and combating of pollution of the sea by oil and other harmful substances; to determine liability in certain respects for loss or damage caused by the discharge of oil from ships, tankers and offshore installations; and to provide for matters connected therewith.”  Although the Marine Pollution (Control and Civil Liability) Act 6 of 1981 provides for the protection of the marine environment from pollution by oil and other marine harmful substances, this Plan focuses specifically on oil only.

The requirements of the Marine Pollution (Control and Civil Liability) Act 6 of 1981 and the South African Maritime Safety Authority Act 5 of 1998 determine the strategy laid out in this Plan and so the Plan delineates the responsibilities of the different parties involved in the national preparedness and response system, including both the public and private sectors.

The fundamental and primary aims of the Plan are to:

(a)	Protect human health and safety;

(b)	Minimise detrimental environmental impacts; and

(c)	Provide for the restoration of the environment, as nearly as is practicable, to pre-spill conditions.

In the event of an incident, a response should be formulated according to the following priorities:

(a)	Human health and safety;

(b)	Natural environment;

(c)	Commercial resources; and

(d)	Amenities

The above priorities could be re-prioritised depending upon the circumstances surrounding the incident. However, safety of life always remains the first priority.

No two incidents are ever the same and therefore the level and intensity of a response varies from incident to incident. One incident may require a major response involving all stakeholders and role players and the next may only require a watching brief by the South African Maritime Safety Authority (SAMSA) and Department of Environment Affairs (DEA).

1.2	Geographic coverage of Plan

The provisions of the Marine Pollution (Control and Civil Liability) Act 6 of 1981 apply to ships and offshore installations within the prohibited area of South Africa.  The prohibited area is defined in the Act as the area encompassing the internal waters, territorial waters and the exclusive economic zone (EEZ) of South Africa and those of the Prince Edward Islands.

The internal waters are waters connected to the sea on the landward side of the base lines as provided for in the Maritime Zones Act 15 of 1994.  Internal waters cover harbours, estuaries and coastal lagoons.  The Republic enjoys full sovereign rights over these internal waters and foreign ships are denied innocent passage in these waters unless they are specifically arriving or departing from a port in the Republic or are in distress.  The most prominent internal water of South Africa is False Bay, having as its base line the line joining Cape Point and Cape Hangklip.

The territorial waters of the Republic are measured seaward 12 nautical miles from the base lines and within this zone  foreign-flagged ships enjoy the right of innocent passage.

The exclusive economic zone extends 200 nautical miles on the seaward side of the base lines. The Republic exercises full sovereign rights over both living and non-living natural marine resources in the EEZ. Furthermore, the Republic, through the Marine Pollution (Control and Civil Liability) Act 6 of 1981 exercises environmental protection jurisdiction in this area.

Therefore, for practical purposes, ships, terminals and offshore installations may not intentionally discharge harmful substances in the internal waters and EEZ of South Africa and the Prince Edward Islands (and Gough Island) in the Southern Ocean.

As far as the high seas are concerned, South Africa has no pollution jurisdiction beyond its EEZ, except as is provided for in the Marine Pollution (Intervention) Act 64 of 1985 and the Maritime Zones Act (1994).  In terms of Act 64 of 1985  the Republic may take action on the high seas when a maritime casualty or act related to the casualty results in or can result in  pollution of the coastline.  Any action the Republic takes shall be necessary to prevent, mitigate or eliminate grave and imminent danger to the coastline from pollution.

1.3	Definitions

“CRU” is the Casualty Response Unit of SAMSA, which is made up of a cross section of SAMSA staff under the control of the Executive Head: Centre for Sea Watch and Response, whoare  on short notice to respond to a situation where  a casualty is causing pollution or threatening to cause pollution.

“Convention”  means  the  International  Convention  of  Civil  Liability  for  Oil  Pollution  Damage,  signed  in Brussels on 29 November 1969 and published for general information under Gazette Notice No. 58 of 1978 in the Government Gazette No. 5867 of 27 January 1978, and includes any amendments thereof and additions thereto signed, ratified or acceded to by the Republic of South Africa.

“DEA” is the Department of Environmental Affairs

“DEA local  coastal oil spill contingency plans” are plans compiled and maintained by DEA which set out the actions to be taken when there is a threat of oil impacting on the shoreline, or where an impact has occurred. The coastline from the Orange River mouth to the Mozambique border is divided into zones, each of which has its own specific local coastal contingency plan.

“Harmful substance” means any substance which, if introduced into the sea, is likely to create a hazard to human health, harm living resources and marine life, damage amenities or interfere with other legitimate uses of the sea, and includes oil and other substances subject to control by MARPOL 1973/78, and mixtures of such substances and water or any other substance.

“Incident” refers to a spill of oil or other harmful substance or a threatened spill of oil or other harmful substance.

“JOC” is the Joint Operation Centre where the JRC meets and where the stakeholders and role players can co-ordinate any activities regarding the incident.

“JRC” is the Joint Response Committee established by SAMSA in the event of a major incident, chaired by SAMSA  and  made  up  of  the  stakeholders  and  role  players  dealing  with  the  prevention  and  combating operations in the event of such an incident.

“Offshore installation” means a facility situated wholly or partly within the prohibited area and which is used for the transfer of harmful substances from a ship or tanker to a point on land or from a point of land to a ship or tanker or from a bunkering vessel to a ship or a tanker, and includes any exploration or production platform situated within the prohibited area and used in prospecting for the mining of natural oil.

“Oil” Oil means petroleum in any form including crude oil, fuel oil, sludge, oil refuse and refined products.

“P&I Club” are the representatives of the ship owner’s protection and indemnity insurance.

“Prohibited area” means the internal waters, the territorial waters and the EEZ and, in relation to an offshore installation, includes the sea within the limits of the continental shelf.

“Role players” are the persons and/or organisations who have a role to play in preventing and combating pollution of the sea by oil.

“Stakeholders” are the persons who may have an interest in any of the activities being undertaken in relation to the incident or who may in any way be impacted by the incident.

“SANCCOB” is the South African National Foundation for the Conservation of Coastal Birds.

“SAMSA” is the South African Maritime Safety Authority established in terms of the South African Maritime Safety Authority Act 5 of 1998, and more specifically the office of the Executive Head: Centre for Sea-watch & Response.

“Sea” means the water and the bed of the sea and includes the land between the high- and low-water marks as well as any tidal lagoon or tidal river as defined in Section 1 of the Sea-shore Act 21 of 1935.

“Tanker” means any seagoing vessel whatsoever, actually carrying oil in bulk as cargo and in respect of which the provisions of the Convention are applicable.

“Terminal” means….. (include definition) any shore-based facility which is used for the transfer of liquid substances from ship to shore or from shore to ship and includes the associated bulk storage tanks.

“Tiers 1, 2 and 3” are categories of response to an incident which require a local, national or international response, with Tier 1 being the lowest category of response.

“Vessel” in the context of this Plan, means any ship, tanker or offshore installation that could in the event of an incident, cause or threaten to cause pollution of the marine environment by a harmful substance.

“Responsible Party”....... include definition) means any person or organisation who has a responsibility for taking any action in the event of an incident.

# REPLACE THE WHOLE OF 1.4 WITH CHAPTER 4 “ACTS AND AGREEMENTS RELATING TO THE COMBATING OF OIL POLLUTION” FROM THE COASTAL PLANS – IT READS BETTER, IS MORE UP TO DATE AND FOR THE SAKE OF CONSISTENCY.

# 1.4 International and National agreements and regulations relating to the Prevention and Combating of Oil Pollution at Sea.

# International and national legislation, relevant to combating and dealing with oil spills, is summarised below.

# 1.4.1	INTERNATIONAL CONVENTIONS (and related South African Legislation)

Below, are the main international conventions to which South Africa is a Party.  For ease of reference, the conventions have been grouped according to the areas to which they are most relevant, namely, oil pollution, operational requirements in respect of oil pollution and compensation for oil pollution damage.  Where applicable, the domestic legislation giving effect to the convention is provided.

4.1.1 	Oil Pollution

United Nations Convention on Law of the Sea (UNCLOS)

UNCLOS is among the conventions ratified by South Africa.  UNCLOS imposes a general obligation on states to protect and preserve the marine environment.  It further provides that states shall take all measures to prevent, reduce and control pollution of the marine environment from any source, using for this purpose the best practicable means at their disposal and in accordance with their capabilities.

States, acting through the competent international organisation or general diplomatic conference are required to establish international rules and standards to prevent, reduce and control pollution of the marine environment from vessels and promote the adoption of routeing systems designed to minimise the threat of accidents which might cause pollution of the marine environment, including the coastline, and pollution damage to the related interests of coastal states.  Such rules and standards are required to be re-examined from time to time as necessary.

1.4.1.2	Compensation for Oil Pollution Damage

International Convention on Civil Liability for Oil Pollution Damage, 1992 (CLC)

This convention (originally adopted in 1969) aims to ensure that adequate compensation is available to persons exposed to oil pollution damage resulting from maritime casualties involving oil-carrying ships.  It applies exclusively to pollution damage caused in the territory, including the territorial sea, of a contracting state and the exclusive economic zone of a contracting state.

It governs the liability of ship owners for oil pollution damage by laying down the principle of strict liability.  The onus is on the owner to prove in each case that any of the exceptions should operate.

It also creates a system of compulsory liability insurance. Ships covered by the convention are required to maintain insurance or other financial security in sums equivalent to the owner’s total liability for one incident.  The ship owner is normally entitled to limit his liability to an amount which is linked to the tonnage of his ship, as stipulated in the Convention.

The convention applies to all sea going vessels actually carrying oil in bulk as cargo but only ships carrying

more than two thousand tons of oil are required to maintain insurance in respect of oil pollution damage.  An owner shall not be entitled to limit his or her liability if it is proved that the pollution damage resulted from his or her personal act or omission, committed with the intent to commit such damage, or recklessly and with knowledge that such damage would probably result. The 1992 CLC was adopted by South Africa in 2004.

Marine Pollution (Control and Civil Liability) Act, 6 of 1981

This Act provides for the protection of the marine environment from pollution by oil and other harmful substances. It goes further than the CLC in that it deals with other harmful substances whereas the CLC is limited to oil pollution damage only.  The Act provides for criminal as well as civil liability following a discharge which causes pollution of the sea.

While the Act is administered by the Department of Transport, the administration of the provisions of the Act regarding the combating of pollution of the sea by oil were assigned to the then Minister of Environmental Affairs and Tourism (now Minister of Water and Environmental Affairs) with effect from 20 May 1986.  Furthermore, many of the administrative functions were transferred to the South African Maritime Safety Authority (SAMSA) in 1998.

An oil spill caused negligently or intentionally falls within the definition of discharge.

The Act sets out the powers of SAMSA to take steps to prevent the pollution of the sea where a harmful substance is being or is likely to be discharged from a ship or a tanker.  Such steps include requiring the master or owner of such ship or tanker to unload the harmful substance from the ship or tanker, to dispose of any harmful substance so unloaded or to move the ship or tanker to a place specified by SAMSA.  Where the master or the owner of a ship or tanker is not capable of complying with such requirements or cannot reasonably be expected to comply with these, SAMSA may cause such steps to be taken.  Furthermore, where any harmful substance is discharged from a ship or tanker, the authority may cause any pollution of the sea caused thereby to be removed.

The owner of any ship, tanker or off-shore installation shall be liable for any loss or damage caused in the area of the Republic by pollution resulting from the discharge of oil, the cost of any measures taken by SAMSA after an incident has occurred for the purposes of reducing loss or damage caused or any loss or damage caused by measures so taken after a discharge has occurred.

The costs referred to shall include an amount deemed by the Director-General to be sufficient to compensate the South African National Foundation for the Conservation of Coastal Birds or any similar organisation approved by the Minister for expenses incurred in treating and rehabilitating coastal birds polluted by oil that has been discharged.

International Convention on the Establishment of an International Fund for Oil Pollution Damage, 1992 (1992 Fund Convention)

This convention establishes the International Oil Pollution Compensation Fund to provide compensation for pollution damage to the extent that the protection afforded by the 1992 CLC is inadequate.  It applies exclusively to pollution damage caused in the territory, including the territorial sea of a contracting state and the exclusive economic zone of a contracting state.

South Africa acceded to the 1992 Fund Protocol which amended the 1972 Convention on the Establishment of an International Fund for Compensation for Oil Pollution Damage and is accordingly 1 of 103 states for which the 1992 Fund Convention is in force.  This Convention has not yet been brought into effect locally.  This means that, in monetary terms, South African claimants would be able to recover no more than R 196 million under the present legislation. Once the enabling legislation is passed, a combined total recoverable amount under the two Conventions (CLC and the Fund) would be approximately R 2,85 billion.  Therefore it is essential that enabling legislation be passed as soon as possible.

1.4.1.3	Operational Requirements

The International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978 relating thereto (MARPOL)

South Africa is a signatory to the MARPOL convention which is the main international convention concerning the prevention of pollution of the marine environment from ships by operational or accidental causes.  The Convention includes regulations aimed at preventing and minimising pollution from ships and contains 6 technical annexes which set out detailed rules and standards.

Annexure 1 contains regulations for the prevention of pollution by oil and is mandatory for state parties.  The discharge into the sea of oil or oily mixtures is prohibited except when certain conditions are satisfied.  The Marine Pollution (Prevention of Pollution from Ships) Act 2 of 1986 (discussed below) incorporates the convention and annexure 1 into South African domestic law.

Marine Pollution (Prevention of Pollution from Ships) Act, 2 of 1986 (the MARPOL Act)

The MARPOL Act gives effect to the MARPOL Convention, by providing for the protection of the sea from pollution by oil and other harmful substances discharged from ships.  This Act is administered by the Department of Transport.

The Act provides for the Minister to make regulations to give effect to the provisions of the Convention.  However this also extends to the Minister making regulations to exempt certain classes of ships from the provisions of the Convention thereby resulting in South Africa not being entirely restricted by the provisions of the Convention.

International Convention on Oil Pollution Preparedness, Response and Cooperation, 1990 (OPRC Convention)

The OPRC Convention is designed to facilitate international cooperation and mutual assistance in preparing for and responding to major oil pollution incidents and to encourage states to develop and maintain an adequate capability to deal with oil pollution emergencies.

Ships flying the flags of contracting states are required to have on board a shipboard oil pollution emergency plan.  Operators of offshore units, authorities or operators in charge of sea ports and oil handling facilities must have oil pollution emergency plans or similar arrangements which are coordinated with the national system for responding promptly and effectively to oil pollution incidents.

Ships are required to comply with the oil pollution reporting procedures and the details of the actions to be taken in this regard are set out in the Convention.

The Convention makes provision for parties to cooperate and provide advisory services, technical support and equipment for the purpose of responding to an oil pollution incident and provision is made for the reimbursement of any assistance provided.  South Africa has signed this convention, but has not yet brought it into effect locally.

# 1.4.2 National Legislation (other than that associated with international conventions)

Constitution of the Republic of South Africa Act, 1996

Section 24 of the Constitution states that everyone has a right to an environment that is not harmful to their health or well-being and to have the environment protected for the benefit of present and future generations through reasonable legislative and other measures that

(i) prevent pollution and ecological degradation;

(ii) promote conservation; and

(iii) secure ecologically sustainable development and use of natural resources while promoting justifiable economic and social development.

The argument may be made that in order to give effect to this right, especially as it relates to preventing pollution of the marine environment, reasonable measures must be taken to combat oil spills as well as measures to remedy the effects of spills.

National Environmental Management Act, 107 of 1998 (NEMA)

NEMA is administered by the Department of Environmental Affairs and provides for cooperative environmental governance by establishing principles for decision-making on matters affecting the environment.

One of the most important principles relevant to the oil spill contingency plan is that the costs of remedying pollution, environmental degradation and consequent adverse health effects, and controlling further pollution, environmental damage or adverse health effects must be paid for by those persons responsible for harming the environment.

Section 30 of NEMA is relevant to oil spills as it deals with the control of emergency incidents.  An emergency incident is an unexpected sudden occurrence leading to serious danger to the public or potentially serious pollution of or detriment to the environment, whether immediate or delayed.

The responsible person must as soon as reasonably practicable after knowledge of the incident, take all reasonable measures to contain and minimize the effects of the incident, undertake clean up procedures, remedy the effects of the incident and assess the immediate and long term effects of the incident on the environment and public health.  A relevant authority (which includes a municipality, a provincial head of department, the Director-General or any other Director-General of a national department) may direct the responsible person to undertake specific measures within a specific time to fulfill his or her obligations in terms of this section.

Should the responsible person fail to comply or inadequately comply with a directive, the relevant authority may take the measures it considers necessary to contain and minimize the effects of the incident, undertake clean-up procedures and remedy the effects of the incident.  The relevant authority may claim reimbursement of all reasonable costs incurred in the taking of such measures from every responsible person jointly and severally.

NEMA provides for a Duty of Care that requires reasonable measures to be taken for the prevention of pollution or environmental degradation. This is particularly relevant in dealing with responsibility for oil spill damage. The National Environmental Management: Integrated Coastal Management Act reaffirms this Duty of Care insofar as it relates to the coastal environment.  The National Water Act, 36 of 1998 also imposes a similar Duty of Care.  Below, we explain the Duty of Care provisions from these acts.

Section 28 of NEMA provides that every person who causes, has caused or may cause significant pollution or degradation of the environment must take reasonable measures to prevent such pollution or degradation from continuing or in so far as such harm to the environment is authorised by law or cannot be reasonably avoided or stopped, to minimize and rectify such pollution or degradation of the environment.

The steps required by this section include the taking of measures to control any activity causing the pollution, preventing the movement of pollutants, eliminating any source of the pollution or remedying the effects of the pollution.

Where a responsible person fails to take the required measures, the Director-General or a provincial head of department may direct the responsible person to commence taking specific

reasonable measures before a given date and to complete them before a specified reasonable date.  If the responsible person fails to comply or inadequately complies with such a directive, the Director-General or provincial head of department responsible for Environmental Affairs may take reasonable measures to remedy the situation or apply to a competent court for appropriate relief.  The Director-General or provincial head of department may recover the costs for reasonable remedial measures to be undertaken before such measures are taken and all costs incurred as a result of applying to court for appropriate relief.

Any person may after giving the Director-General or provincial head of department thirty days notice, apply to a competent court for an order directing the Director-General or any provincial head of department to take specific measures for the protection of the environment if the Director-General or provincial head of department fails to inform such person in writing that he or she has directed the person to take such steps.

National Water Act, 36 of 1998

The National Water Act deals with pollution of water resources and also provides for the control of emergency incidents.  Following an emergency incident such as an accident involving the spilling of a harmful substance that finds or may find its way into a water resource (water resource is defined to include a watercourse, surface water, estuary or aquifer), the responsibility for remedying the situation rests with the person responsible for the incident or the substance involved.

Measures to be taken by such person include taking all reasonable measures to contain and minimise the effects of the incident, undertaking clean-up procedures and remedying the effects of the incident.  Where such person fails to act, the relevant catchment management agency may take the necessary steps and recover the costs from every responsible person.  The Act is administered by the Department of Water Affairs.

The National Water Act also provides a duty of care similar to that in NEMA.  This however deals more specifically with situations where pollution of a water resource occurs as a result of activities on land.

National Environmental Management: Integrated Coastal Management Act, 24 of 2008

Included among the aims of the Act is the control of dumping at sea and pollution in the coastal zone.  The Act provides that in fulfilling the rights contained in section 24 of the Constitution, the state through its functionaries and institutions implementing the Act, must act as a trustee of the coastal zone and must in implementing the Act take reasonable measures to achieve the progressive realisation of those rights in the interest of every person.

The state in its capacity as the public trustee of all coastal public property must take whatever reasonable legislative and other measures it considers necessary to conserve and protect coastal public property for the benefit of present and future generations.

The Minister, an MEC or a municipality concerned may institute legal proceedings or take other appropriate measures to prevent damage or recover damages for harm suffered to coastal public property or the coastal environment or to abate nuisances affecting the right of the public in its use and enjoyment of coastal public property.  Accordingly, these provisions can extend to damage caused or measures taken to prevent such damage as a result of oil spills.

This Act also provides that section 28 of NEMA applies to any impact caused by any person that has an adverse effect on the coastal environment.  The persons to whom section 28 of NEMA apply include any person who produced or discharged a substance which caused, is causing or likely to cause an adverse effect and this may therefore include discharges from ships.

South African Maritime Safety Authority Act, 5 of 1998

This Act provides for the establishment of SAMSA whose objectives are to ensure the safety of life and property at sea, to prevent and combat pollution of the marine environment by ships and to promote the Republic’s maritime interests.

SAMSA may perform a function itself, in co-operating with another person or by delegating or assigning the power or duty concerned to another person.  “Person” includes the state, a province, the government or an agency of the government of a foreign country or any juristic or natural person.

Certain functions of SAMSA are performed by the Department of Environmental Affairs.  The responsibility for matters relating to the combating of pollution mentioned in Marine Notice No. 2 of 1996 issued by the Department of Transport on 24 January 1996 is regarded as having been assigned to the Department Environmental Affairs by this Act.

The National Health Act, 61 of 2003 (Replaces the Health Act, 63 of 1977)

The National Health Act provides that every local authority shall take all lawful, necessary and reasonably practicable measures to prevent the occurrence, within its district, of any condition which could be harmful or dangerous to the health of any person within its district or the district of any other local authority.  Where such nuisance or condition has occurred, the authority must take measures to abate or remedy such condition.  Accordingly where an oil spill could be harmful or dangerous to human health, the local authority may take measures to remedy its effects.

The Sea-Shore Act, 21 of 1935

The Sea-Shore Act makes specific provision for the protection of public health.  The competent authority to whom the administration of the Health Act has been assigned, may declare that any local authority may exercise, in respect of the sea-shore and the sea situated within its area of jurisdiction or adjoining such area, any of the powers which are conferred by the Health Act on a local authority.  This could therefore be extended to include the taking of measures to remedy an oil spill in cases where it could be harmful to human health.

The Minister of Transport may, in terms of this Act, make regulations or authorise any local authority to make regulations concerning the prevention or the regulation of the depositing or the discharging upon the sea-shore or in the sea of offal, rubbish or anything liable to be a nuisance or danger to health.

The Act will be repealed by section 98 of the National Environmental Management: Integrated Coastal Management Act when that section comes into force.

Other Relevant Legislation

The following legislation while not directly applicable in dealing with measures to be taken in cases of oil spills is still of relevance for purposes of the protection of the marine environment from oil spills.

Merchant Shipping Act, 57 of 1951

SAMSA is responsible for the administration of this Act.  The Act imposes an obligation on an owner of a ship to secure the sea worthiness of a ship.  Unseaworthy ships may be detained and where any ship is detained, it may be inspected by a surveyor who shall report on any supposed defects or deficiencies.

Dumping at Sea Control Act, 73 of 1980

This Act brings into force domestically the provisions of the Convention on the Prevention of Marine Pollution by Dumping of Wastes and Other Matter, 1972 (the London Convention).  It provides for the control of dumping at sea of various substances and structures, including vessels, platforms or other man-made structures.

South Africa is a signatory to the 1996 Protocol, which will eventually replace the current Convention. The Protocol introduces the precautionary and polluter pays principles and expands the objectives of the Convention to include the elimination of pollution where practicable.

The Act is administered by Department of Environmental Affairs.  It will however be repealed by section 98 of the National Environmental Management: Integrated Coastal Management Act when that section comes into force.

Marine Pollution (Intervention) Act 64, of 1987:

This Act gives domestic effect to both the Intervention Convention Relating to the Intervention on the High Seas in Cases of Oil Pollution Casualties, 1969 as well as the Protocol Relating to Intervention on the High Seas in Cases of Marine Pollution by Substances other than Oil, 1973.  It allows the Minister of Transport to make regulations to give effect to the provisions of the Convention and the Protocol.

Disaster Management Act, 57 of 2002

This Act provides for, among others, an integrated and co-ordinated disaster management policy that focuses on preventing or reducing the risks of disasters, mitigating the severity of disasters, emergency preparedness, rapid and effective response to disasters and post-disaster recovery.

Disaster is defined in section 1 of the Act as “Is of a magnitude that exceeds the ability of those affected by the disaster to cope with its effects using only their own resources” and could presumably include oil spills. The Act not only addresses the occurrence of a disaster, but also makes provision for the “threat of” a disaster, i.e. potential disaster.

The Act does not however apply to a disaster where such occurrence can be dealt with effectively in terms of other national legislation aimed at reducing the risk, and addressing the consequences of occurrences of that nature and identified by the Minister by notice in the Gazette.  Accordingly, this Act would not be of direct application in cases of oil spills as the other Acts related to oil pollution response, provide measures that can be enforced in dealing with oil spills.

The Public Finance Management Act, 1 of 1999

This Act regulates financial management in the national government and provincial governments and ensures that all revenue, expenditure, assets and liabilities of these governments are managed efficiently and effectively.  It provides for the establishment of a national treasury consisting of the Minister who is the head of treasury and the national department or departments responsible for financial and fiscal matters.

While the Act is not directly relevant to procedures in the event of oil spills, the Minister or MEC for finance in a province may authorize the use of funds from the respective national revenue fund or provinces provincial revenue fund respectively to defray expenditure of an exceptional nature which is not currently provided for and which cannot, without serious prejudice to the public interests be postponed to a future appropriation of funds.  Thus in particular circumstances additional budget could possibly be allocated in cases of oil spills that threaten the public interests.

Municipal Finance Management Act, 56 of 2003

All expenditure by municipalities is regulated by this Act.

National Ports Act, 12 of 2005

This Act relates to the establishment of the National Ports Authority and the Ports Regulator; to provide for the administration of certain ports by the National Ports Authority; and to provide for matters connected therewith. The Authority is required in the performance of its functions to ensure that a fair and reasonable balance is achieved between the protection of the environment and the establishment, development and maintenance of ports.

PART II: POLICY AND PLANNING

2	Organisational arrangements

2.1	National Organisations (SAMSA and DEA)

One of the SAMSA objectives listed in Section 3 of the South African Maritime Safety Authority Act 5 of 1998 (the SAMSA Act) is to prevent and combat pollution of the marine environment by ships. Section 52 of the Act assigns the combat function to DEA.

This arrangement of split responsibility was established on 1 October 1985 when DEA was made responsible for matters relating to the combating of oil pollution with the then Department of Transport (DOT) retaining responsibility for prevention of pollution incidents.  The arrangement applied to functions in terms of the Prevention and Combating of Pollution of the Sea by Oil Act 6 of 1981 (as it was originally called).  With the establishment of SAMSA on 1 April 1998, the administration of this Act (now called the Marine Pollution (Control and Civil Liability Act 6 of 1981) was transferred to SAMSA in terms of the schedule to the SAMSA Act.  Section 52 of the SAMSA Act provides for  DEA's continued responsibility for combating oil pollution by assigning the function to them.

Therefore, SAMSA, as the legally designated party responsible for preventing and combating pollution will, in the context of this Plan, be the lead agency and have overall responsibility for responding to pollution incidents from ships.  SAMSA will also be responsible for the overall implementation of the Plan.

SAMSA and DEA are the two stakeholders responsible for pollution prevention and combating.

2.2	Responsibilities of SAMSA and DEA

Because of the assignment of the oil pollution combating functions to DEA, the prevention and combating responsibilities have been split with SAMSA being responsible for the following functions relating to the prevention of oil pollution:

	Overall charge of the co-ordination of the prevention and combating of an incident:

	Control of the technical aspects of shipping casualties;

	Supervision of  oil transhipments;

	Prosecution of  parties guilty of the deliberate discharges of oil;

	Compilation of  contingency plans relating to the control of shipping casualties or potential casualties;

	Administration of  the Acts relating to oil pollution;

	Overall  charge of the legal and financial aspects relating to oil spill incidents and casualties;

	Control of the use of the standby oil pollution prevention tug; and

	Issuing  pollution safety certificates for offshore installations.

And

DEA being responsible for the following functions relating to the  combating oil spills:

- Co-ordination  and implementation of coastal  protection and clean-up measures;

- Control of the use of the pollution combating vessels and surveillance aircraft;

- Control of the use of oil spill dispersants;

- Maintenance  and supply of oil dispersant stocks and certain other oil spill equipment;

- Compilation and maintenance of   the DEA local coastal oil spill contingency plans  and

- Approve, in consultation with SAMSA, the contingency plans for  offshore installations.

The above-mentioned responsibilities  do not limit any additional duties and/or functions which the  DEA may be responsible for under other environmental legislation.

The Provincial Departments of Environmental Affairs shall assist:

- The DEA in updating the local  coastal oil spill contingency plans;

	In building capacity and awareness in the local authorities; and

- Local authorities in the implementation of the DEA local coastal contingency plans.

2.3	General application of the division between DEA and SAMSA

The above split in responsibilities has led to the following division in response to an incident:

The pollution prevention response:

SAMSA, as the lead agency, will, as soon as it obtains knowledge of an incident, intervene to ensure steps are taken to prevent pollution occurring.  This is not always possible as some casualties are catastrophic and nothing done will prevent pollutants entering the sea.  However, it is SAMSA’s policy that the first intervention will be that which will endeavour to keep the pollutants contained in the ship until the ship can be physically removed to a safe place.  Alternatively, when the pollutants have been effectively contained, they can be transferred to another ship or means of containment.

SAMSA manages on behalf of the Department of Transport a deep-sea salvage tug stationed permanently on the coast.  This is South Africa’s first line of defence in preventing pollution and the tug is on permanent standby  to tow  a ship that is polluting or threatening to cause pollution, away from the coast..  As DEA and SAMSA are both responsible for pollution prevention and combating functions , SAMSA will, in the case of a pollution prevention operation only, keep DEA closely informed as DEA and other role players could be affected.

The pollution combating response:

DEA is responsible for co-ordinating the response actions to contain and clean-up pollution that has been released to the sea from the ship.  In co-ordinating these combating actions, DEA’s role is to ensure that all actions taken do not result in a greater environmental, social or economic impact than would have been the case should no action to combat the spill have been taken.

DEA will, in consultation with SAMSA, initiate, approve, oversee and co-ordinate all activities taken by other role-players in responding to the containment and clean-up action.  This is to ensure that all response activities are carried out in a co-ordinated manner and that they result in a net environmental benefit.

2.4	Finance and Compensation

A fund, called the Maritime Fund, has been  established under the control of the Minister.  SAMSA administers the Fund in the prescribed manner and the accounts relating to the Fund are  audited annually by the Auditor-General.  It is not clear here what this fund is used for – it needs to be spelt out here – mention is made below that there is no contingency fund.

Money in the Fund may be applied only for the purpose of furthering the objectives of the Authority referred to in section 3.

Money in the Fund that is not required for immediate use is  invested with a bank registered under the Bank Act, 1990(Act 94 of 1990).

Money in the Fund at the end of any financial year is  carried forward as a credit to the Fund for  the ensuing financial year.  The Chief Executive Officer (is this the CEO of SAMSA or someone else – need to be specific) is the accounting officer to the Fund charged with the responsibility of accounting for money received and expenditure incurred by the Fund.

South Africa has no dedicated State pollution contingency fund. The State’s policy regarding costs or losses arising from an incident is that the polluter pays. Where the polluter cannot be identified, the State has to bear the cost of the clean-up.

In any incident where there is a ship involved, the owner could impose a limit on the funds available for costs incurred in the prevention and combating roles.  The ship owner can, in terms of the provisions  of the liability Convention (specify) or the Merchant Shipping Act (specify) limit his or her liability.  This limitation of liability is a major deciding factor on the amount of money available for the prevention and clean-up activities.  Where the polluter cannot be identified, the State is responsible for the costs incurred and this would  be a major factor in deciding how much money is available for combating operations.

In a tanker incident, the State has recourse to the Fund (International Oil Pollution Compensation Fund, 1992) for compensation to be paid in excess of the liability limit provided for in the Marine Pollution (Control and Civil Liability) Act 6 of 1981.

There is no limitation applicable to an incident resulting from an offshore installation.

Initially, because the Republic has no dedicated State pollution contingency fund, the costs of such operations fall to those involved in the operations and in line with the “polluter pays” principle may subsequently be claimed as costs and damages from the owner of the ship.

It is however accepted that small service providers cannot be expected to carry the costs of providing services to the State for any length of time and SAMSA, as the responsible authority, will assist in enabling a response to get under way, by way of underwriting such actions as it considers necessary in the early phases of any response.  This undertaking is severely limited and a better arrangement regarding underwriting the costs will have to be made in the event. WHAT DOES THIS LAST SENTENCE MEAN – CLARITY IS NEEDED.

In many cases, the owner will be  identified and have P&I Club insurance cover, making the recovery of costs and  damages a strong probability, especially if the claims are reasonable and any expenditure has been properly audited and controlled.  It is important to bear in mind the limitation of liability regime that is in place in the Republic.  EXPLAIN THIS ie the limit that the current Act imposes Note that this will change when the new Bill is enacted by the end of the year.

It is quite possible however that there might be no response from the owner and any interventions and clean-up must be carried out nonetheless.  These actions need to be tempered however with the reality that these costs may not be recoverable from the owner and that the State would need to be approached for compensation.  SAMSA will make every effort to secure assets or funds associated with the owner in cases where the owner is not accepting responsibility for the costs incurred.

An independent auditor should be designated at a very early stage of the incident so that there is no confusion as to the accuracy of claims or exactly what has to be compensated for.  He/she should keep records and audit actions approved by the JRC (SAMSA) during the operation.  These records will be required to support the claims for cost recovery and to show that any action taken was reasonable  and without which the operation would not have been a success.

Reimbursement of costs incurred shall be made following procedures as set out in international conventions (SPECIFY) and according to the suggestions of an independent auditor.   REPEAT.

2.5	The Joint Response Committee (JRC)

In the event of a major incident, SAMSA will bring together a JRC, made up of role players, the function of which will be to consult, inform, and co-ordinate activities in dealing with the incident.  The stakeholders in the JRC are SAMSA,DEA and other authorities with a role to play during the incident. (see definitions)  Other stakeholders, such as the shipowner’s representative, his insurers, conservation bodies, etc may from time to time be invited to attend JRC meetings.  The ship’s representatives or insurers should be kept informed of the reasons for all actions being taken during the incident to so as to minimise the possibilities of a dispute regarding the costs incurred. It is important that those paying the costs are part of the decision-making process in order to ensure their support for the actions being taken and the costs incurred. The independent auditor will ensure that proper records of decision are kept throughout the incident.

.

In any prevention or combating operation there is a limit on the budget available for costs incurred and the JRC will agree to an activity keeping in mind the priority of the activity and the balance of funds available in the budget.

SAMSA as the agency responsible for the implementation of the requirements of the Marine Pollution (Control and Civil Liability) Act 6 of 1981 has  the right  to veto any activity agreed to by the JRC or any activity agreed to unilaterally by any other stakeholder or role player.

2.6	Role Players and Stakeholdars

The following table summarises the roles and responsibilities of national, provincial and local authorities, as well as other stakeholders who may have an interest in the pollution incident.

Any of the organisations below may be called upon to serve on the JRC, depending on the scale and specific circumstances of the incident.

2.6.1	National Authorities

| Organisation |  | Area of jurisdiction |  | Roles and responsibilities |
| --- | --- | --- | --- | --- |
| SAMSA |  | The prohibited area |  | - Preventing and combating pollution of the sea (See 2.1 – 2.3). - Combating of oil pollution assigned to DEA. Retain responsibility for preventing pollution ie keeping the oil within the ship. |
| DEA |  | The prohibited area |  | - Co-ordinating the combating of oil pollution (ie shoreline protection and clean up) once the oil has escaped from the ship. - Licensing of oily waste disposal |
| Department of Water Affairs |  | Soil and groundwater |  | - Provide specialist advice in respect of oily waste disposal |
| SA National Defence Force |  | The prohibited and land area |  | - Provide assistance with transport , manpower and aviation and maritime services as the need arises during an incident. |
| SA Police Services |  | Internal waters and land area |  | - Provide support in the investigation as to the cause of the incident. - Enforce the recommendations of the SAMSA investigation unit. - Provide manpower for crowd control and protection of property. |
| SA Weather Service | National |  | - Provide weather forecasts and weather history as required for pollution prevention and combating operations. |  |
| SARS – Customs and Excise | National |  | - Facilitate entry of oil spill response equipment provided through international assistance or contract. - Facilitate the management of salvaged goods, including payment of duties - Assist with customs clearance for disposal of pollutants and wreck |  |
| Department of Home Affairs | National |  | - Facilitate the entry of personnel provided through international assistance or contract - Provide assistance with the repatriation of distressed seamen. |  |
| SA National Parks | Coastal areas falling within national parks. |  | - Restrict or relocate wildlife to prevent them becoming oiled in areas under their jurisdiction. - Facilitate the capture and rehabilitation of oiled wildlife in areas under their jurisdiction. |  |
| Transnet National Ports Authority | Port areas |  | - Prevent, manage, co-ordinate and clean up oil spillages within Port areas under direction of DEA - Escalate function to SAMSA/DEA if scale of response exceeds own resources and adjacent areas likely to be impacted. |  |
| Department of Agriculture, Forestry & Fisheries |  |  | - Manage protection and clean up operations within fishing harbours, under the direction of DEA. |  |

2.6.2 Provincial and local authorities

| Organisation |  | Area of jurisdiction |  | Roles and responsibilities |
| --- | --- | --- | --- | --- |
| Provincial Departments of Environment ie N Cape, W Cape, E Cape and KZN. | Coastal areas within the Province | - Organise clean up operations and provide assistance with response activities in terms of the local oil spill contingency plans, under the direction of DEA. - Assist local authorities where capacity is limited - Assist in sourcing local labour and equipment. - Provide specialist advice with respect to conservation of natural resources, waste disposal, and other relevant local issues. - Assist with crowd and traffic control |
| Provincial Nature Reserves |  | - Restrict or relocate wildlife in conservation areas under their jurisdiction to prevent them becoming oiled. - Facilitate the capture and rehabilitation of oiled wildlife in areas under their jurisdiction. - Provide specialist advice on conservation issues. |
| Local authorities eg Municipalities, City Metro’s, etc |  | Coastal areas within municipal / metro boundaries. |  | - Provide assistance with response activities in terms of the local oil spill contingency plans, under the direction of DEA. - Assist in sourcing local labour and equipment. - Assist with crowd and traffic control |
| Disaster Management Units (Provincial and Municipal) |  | Provincial or Municipal areas |  | - Assist in the manning of a JOC - Provide support services to the operational functions. |

2.6.3 Stakeholders and other interested bodies

| Organisation |  | Function |
| --- | --- | --- |
| Ship Owner | - Inform SAMSA of the incident. - Take all steps to minimise pollution or threat of pollution. - Appoint on-site representative - Representative to participate in JRC activities. - If applicable, appoint salvors. - Ensure involvement of P&I Club or Underwriters (insurers) |
| Smit Amandla | - On instruction from SAMSA, mobilise salvage tug as a first line of defence in preventing a pollution incident - On instruction from DEA, pollution abatement vessels for “at sea” response. |
| Salvors | - Under contract to the owner ( or alternatively under instruction from SAMSA) salvage the ship and/or take the necessary action to prevent or minimise pollution from the casualty |
| International Tanker Owners Federation (ITOPF) | - ITOPF provides technical services to its tanker owner members and others in relation to ship-source spills of oil or chemicals, including advice on response, damage assessment and analysis of claims for compensation. - ITOPF may be requested by the shipowner or the insurers to attend the scene of the incident to provide the assistance mentioned above. |
| South African Petroleum Industry Association (SAPIA) |  | - In the event of an incident involving a local oil company’s vessel or cargo or an oil oil spill from a company facility: - Facilitate communication between the oil company and the regulator - Provide personnel and equipment to respond to the incident, under the direction of the DEA. - Assist with any aspect of the response, as requested by DEA or SAMSA. |
| South African National Foundation for the Conservation of Coastal Birds (SANCC0B) | - Participate, together with provincial authorities, in the rescue, transport, cleaning and rehabilitation of oiled seabirds. - Provide specialist advice on the protection of seabirds during an oil spill incident - Co-ordinate the efforts of other NGO’s in similar work. |
| National and international oil spill response service providers |  | - Provide oil spill response equipment, materials and specialist manpower, under contact to SAMSA/DEA, to contain, recover and dispose of spilt oil. |

2.7	Geographical Responsibility

Bi-lateral and multi-lateral agreements ARE THERE ANY IN PLACE? OR IS THIS REFERRING TO THE NAIROBI AND ABIDJAN CONVENTIONS?

2.8	International assistance

In terms of the International Convention on Oil Pollution Preparedness, Response and Co-operation, 1990 (OPRC, 90),  Contracting Parties (South Africa being one) are obliged to co-operate and assist each other in the following manner:

- Parties agree that, subject to their capabilities and the availability of relevant resources, they will co-operate and provide advisory services, technical support and equipment for the purpose of responding to an oil pollution incident, when the severity of such incident so justifies, upon the request of any Party affected or likely to be affected.

The financing of the costs for such assistance shall be based on the provisions set out in the Annex to this Convention.

- A Party which has requested assistance may ask the International Maritime Organization to assist in identifying sources of provisional financing of the costs referred to in paragraph (1).

- In accordance with applicable international agreements, each Party shall take necessary legal or administrative measures to facilitate:

- the arrival and utilization in and departure from its territory of ships, aircraft and other modes of transport engaged in responding to an oil pollution incident or transporting personnel, cargoes, materials and equipment required to deal with such an incident; and

- the expeditious movement into, through, and out of its territory of personnel, cargoes, materials and equipment referred to in subparagraph (a).

Hence, in terms of this Convention, South Africa may request assistance with oil spill response activities from other countries that have ratified the Convention and similarly, other Contracting Parties may request such assistance from South Africa.

PART III: PREVENTION STRATEGY

3	Pollution Prevention

3.1	Casualty management

SAMSA’s strategy in respect of casualty response can be described as “risk based casualty management”. Ship casualties are an unfortunate and inevitable side effect of sea-based trade and as both the largest trading country on the sub-continent as well as being a strategically placed coastal trade State (geographically), South Africa can expect more than its fair share of ship casualties.  Risk based casualty management is based on a practical real-time evaluation of the benefits versus the risks, as the event unfolds, and where necessary and possible, active intervention.

IN TERMS OF SOUTH AFRICAN MARINE POLLUTION LEGISLATION AND THE WRECK AND SALVAGE ACT, SAMSA HAS THE CLEAR AUTHORITY TO TAKE ANY ACTION DEEMED NECESSARY TO PREVENT POLLUTION OF THE MARINE ENVIRONMENT, AND THIS INCLUDES THE AUTHORITY TO OVERRIDE A SALVOR’S DECISION.

SAMSA’s priorities in managing a casualty are:

a)	Safety of life (saving the lives of people on board or otherwise threatened by the casualty);

b)	The preservation of the vessel, or removal of the vessel from the shore, with harmful substances contained and intact, in order to prevent pollution;

c)	The removal of oil and other harmful substances, by the most practical means, from the vessel to prevent pollution, should the option in (b) above fail;

d)	The preservation of property (coastal properties, cargo and/or ship and

e)	The removal of the wreck.

It is envisaged, and it is indeed current practice (after considering, inter alia, the risks of pollution and financial implications presented by any particular damaged vessel) for SAMSA to allow the vessel to carry out repairs in the South African territorial waters or even carry out cargo transfers (under supervision) in a designated bay, where the benefits far outweigh the risk profile. (See Places of Refuge in paragraph 3.2).

Benefits in this context also mean assisting a vessel while there is opportunity and means, as opposed to dumping a developing situation on a neighbouring State that may not be afforded the same opportunity and may not even have the means.  A major pollution incident in neighbouring States has a possibility and is almost certain to affect South Africa in due course.  Where there is an imminent or possible threat of such an incident, SAMSA will engage with DEA and other relevant authorities in the process of assessing the risk profile.  If necessary, this will be via the JRC.

SAMSA’s strategy is therefore to ensure that its various legislated responsibilities and duties are carried out. In effect, this means that whenever there is a spill of oil or any other harmful substance from a ship or offshore installation, SAMSA will ensure that:

a)	There is an effective initial response, including communication with DEA;

b)	That there is effective communication and co-operation with affected authorities and parties by way of a JRC;

c)	That the methods and costs of carrying out the combating are in accordance with international practice and the Public Finance Management Act 1 of 1999;

d)	That costs and expenses of such combating are audited throughout.  This is in order to expedite any subsequent claims made in terms of Act 6 of 1981; and

e)	That State expenditure in respect of pollution combating is recovered from the owner in the most expeditious manner.

[Note; In the case of offshore installations, an acceptable, effective and practical contingency plan, as well as the acceptance of direct financial responsibility for any clean-up, has already become part of the procedure for obtaining the statutory Pollution Safety Certificate from SAMSA]

3.2	Places of Refuge

South Africa has, for a number of years, allowed ships in distress to seek a place of refuge along the South African coastline.  As no two incidents are the same, each request is considered on its merits and as a result SAMSA has developed the following policy principles:

a)	The first place of refuge considered is alongside a quay in a port in South Africa.  However, this is subject to the approval of the Transnet National Ports Authority (TNPA) which has its own safety, financial and commercial responsibilities and may well refuse entry to a ship in distress.  Consultation between SAMSA, DEA and the harbour master of the port is a requirement for a ship that seeks refuge in a port; alternatively, practice has shown that places of refuge are limited to Algoa Bay, False Bay and on occasion, St Helena Bay and Saldanha Bay;

b)	There will be consultation between SAMSA and DEA as to the best place to moor the ship taking  into account the environmental impact that might result from her sinking;

c)	Disclosure of all the relevant facts is expected from the owner/operator.  Failure by the owner/operator to disclose all the facts or to participate in an open manner with SAMSA and any other relevant authority will of course produce a negative response to the request for a place of refuge;

d)	A guarantee to cover losses and costs in the event of a pollution incident or for a wreck removal may be required. In any event, insurance cover is a prerequisite;

e)	If the vessel has suffered structural damage or failure, SAMSA will require the owner to enter into a salvage agreement with a local salvor to deliver the vessel to a safe place;

f)	The owner/operator will be expected to place on standby a suitably powered salvage tug with the appropriate towing gear and expertise;

g)	In considering the request, SAMSA, in order of priority, will give consideration to the following:

	Safety of the crew;

	Pollution prevention;

	Type of cargo; and

	The value of the ship and her cargo;

h)	SAMSA may establish a JRC which will function in a pollution prevention capacity; and

i)	SAMSA has the authority to order the removal of the distressed ship at any time it considers it   necessary.

3.3	Reporting and Alerting

Marine Notice No. 16 of 2007 as may be amended from time to time, deals with the reporting of spills and shipping casualties, which may be reported to SAMSA, DEA or any contracted tug at the time.  The initial report may come from any number of sources and be directed to whomever that source deems fit i.e. SAMSA or DEA.  The notice and any subsequent amendment can be found on the SAMSA website under Marine Notices and more specifically under the title of “Reporting of oil spills and shipping casualties threatening pollution”.

Any SAMSA employee who receives a report, hears or overhears any communication which indicates a possible incident (whether a spill or  a ship with problems) is duty  bound to ensure that the information is transmitted  to  the  Operations  Manager,  or  his  stand  in.  The said officer must ensure that the report is investigated until he is reasonably certain that the report or rumour is substantiated and verified or repudiated.

3.4	SAMSA’S action plan

The Executive Head CSWR on notification of the incident, will take all possible steps to confirm the incident.  In addition, wherever practicable and subject to safety and logistical considerations, he shall send a surveyor or a Principal Officer out to the vessel or the site of the incident to make an on-site inspection and a report.  The first-hand information received from qualified persons on site is critical to any reasonable  risk assessment process.

The on-site inspection will be delegated to experienced ship surveyors.  It is SAMSA’s responsibility to allocate a budget for airfares, helicopter hire, hotels, etc., to facilitate the intervention by surveyors and other officers whenever there is a reported incident.

In addition, the SAMSA Casualty Response Unit (CRU) team shall be tasked, as the Operations Manager determines, with carrying out among others the following actions:

a)	Determining the name and contact details of the vessel’s owner;

b)	Inquiring into the insurance status of the vessel and the name of the P&I Club, if any;

c)	Requesting details of the fuel oils and other harmful substances on board;

d)	Evaluating the specific threat of the pollution posed by the vessel in respect of her bunkers and/or cargo on board;

e)	Obtaining any additional particulars, calculations or considerations required by the Operations Manager;

f)	Convening a JRC and establishing a JOC with communications and facilities;

g)	Obtaining a cargo manifest for the ship;

h)	Preparing press releases;

i)	Informing affected authorities and parties; and

j)	Where appropriate, representing SAMSA as part of any active intervention aboard the ship casualty.

3.5	SAMSA casualty response unit

The Unit consists of:

a)	Executive Head CSWR, or appointed official;

b)	Four senior surveyors;

c)	Legal officer;

d)	Public relations officer;

e)	Naval architect; and

f)	An administration officer

The officers are volunteers from the existing cadre.  They are available at short notice.  They are prepared for casualties, and are instructed and carry out exercises periodically in preparation for emergencies and interventions.

3.6	The Joint Response Committee

The JRC is established on the instruction of the SAMSA Operations Manager and it usually comes into operation after consultation between the major role players.

The JRC is chaired by SAMSA’s Executive Head CSWR or a senior SAMSA staff member who is appointed to the chair in writing by the Executive Head CSWR.  His or her function is to bring together all the major role players to discuss, instruct, inform, formulate and agree to plans on preventing and combating the pollution.  SAMSA’s participation in the JRC is closely linked to its CRU.

Participation in a JRC is important.  It ensures a  co-ordinated  operation  in  dealing  with  the  situation.  It provides a forum where concerns can be tabled, discussed and then be allocated to the correct authority to deal with it.  It ensures that action taken is agreed to by the major role players, which in turn, ensures prompt payment for services rendered. Pollution scenarios change all the time and a JRC allows the role players to stay on top of the changing situation.

Not every incident will require the establishment of a JRC.  However, for any incident where the committee is not established, a Principal Officer or a surveyor appointed by a Principal Officer will maintain a watching brief over the incident and resultant clean up.

A JRC will meet regularly, usually daily, sometimes twice a day depending on the incident.  It is important to keep in mind that the time of the role players must not be consumed by committees. As the urgency about an incident decreases, the frequency of the JRC meetings may be reduced.

SAMSA has the responsibility of briefing the press.  An advantage of a JRC is its ability to provide a co- ordinated and factual response to the media.  Practice has shown a combined daily press release from the stakeholders works very well.  In addition, specific stakeholders and role players could be (with their consent) briefed to meet with the press in a media conference. (See paragraph 5 for “Media Response”.)

The JRC will meet at the most convenient disaster management JOC or alternatively in a location which is most convenient to the incident. It is the duty of the officer in the CRU who is tasked with the function to ensure that the JRC has the appropriate communication systems available. To this end SAMSA has available a budget to deal with these expenses if necessary.

Logistics

A rapid response is essential for effective spill clean-up.  Effective logistics ensure the expedient and efficient mobilization of resources and sustained resource availability throughout the response effort.  Some key logistics elements that must be considered by the CRU are listed below:

- Contracts for goods and services

- Shipping and receiving

- Warehousing & staging

- Equipment tracking

- Transportation of goods

- Permits

- Waste management

- Assembly and fabrication

- Demobilization

- Decontamination

- Security

The logistics team is to further:

- Prioritise objectives and allocate resources accordingly

- Allocate equipment appropriate to the job

- Use fixed-wing aircraft for heavy load and extended flights

- Use locally available aircraft when possible

- Locate fuel depots near work areas

- Acquire marine charts ,tide tables, maps, weather forecasts, aviation charts

- Obtain procurement documents, purchase orders ,contract and rental agreements

- Acquire relevant permits

  - Hazardous goods transport

  - Industrial waste disposal

  - In-situ burning

  - Chemical applications ( dispersant, shoreline cleaners)

  - Vessel docking

  - Aircraft

  - Land or property access

  - Approvals for various activities, e.g., washing of booms.

Financial Controls

- Do not make long term agreements

- Document all transactions and agreements

- Appoint expenditure officers and delegate signing authority

- Control inventory of equipment and supplies

- Track purchase orders , invoices ,contracts and labour expenses

3.7	Termination of the Response

SAMSA, as the lead agency, has the function of terminating the response and it will only do this when it is satisfied that effective completion of the response is achieved, based on expert advice.  This advice will  be obtained in consultation with the major role players.  The JRC provides the platform for this consultation to take place and to ensure that all parties have signed off with regard to their various responsibilities.  SAMSA will be responsible for announcing the termination of the response.

Ensure that response debriefing and evaluation is conducted before termination of response.

PART IV COMBATING STRATEGY

4.	Combating Strategy

4.1	Levels of Response

In the Plan a combating response shall be organised according to incidents which are oil or chemical or hazardous substances spills from a casualty or from a bunker or cargo or oil related activity associated with ship operations at a terminal, a refinery or around waterside storage tanks, etc.  These incidents result in the following categories of response:

- A Tier 1 response is where the containment, clean-up and rescue of contaminated fauna can be dealt with within the boundaries of the vessel, berth or a small geographical area where the incident has no impact outside the operational area but poses a potential emergency condition.  Such an incident is a small spill that can be contained and cleaned-up by the ship, terminal or port authority staff using their own resources.  The most common type of Tier 1 response deals with an incident occasioned during a  ship bunkering operation.

- A Tier 2 response is where the nature of the incident puts it beyond the containment, clean-up and rescue of contaminated fauna capabilities of the ship or terminal operator and the containment or clean-up requires the use of some of or all the government and industry resources.  It could be near or some distance from operational centres.  The incident is usually associated with shipping activities in ports or harbours, estuaries, coastal waters, pipelines, tank failures or near shore explorations and production operations.

- A Tier 3 response is where the nature of the incident puts it beyond containment, clean-up and rescue of contaminated fauna capabilities of a national or regional response.  It is usually a large spill which has the probability of causing severe environmental and human health problems.  The response will require assistance from other African States and/or specialised overseas response organisations.  Such incident becomes a major international affair involving almost every aspect of government.  When responding to an incident of this nature, only strategies outlined in the Plan should be engaged.

4.2	Risk Assessment

SAMSA and DEA will continue considering and assessing the risk factors in the South African marine environment.  Risk assessments as and when carried out should be used to update the Plan.

4.3	Protection Priorities

South Africa’s coast line is surrounded by high energy seas. In the majority of incidents regarding oil, these high energy seas disperses the oil so reducing the need for a large commitment of oil retrieving equipment held on standby.  Therefore, the procedure for combating a slick is often a monitoring one.  However if the slick should threaten a high risk estuary area, South Africa is geared to closing the estuary with booms or man-made sand bars.

In responding to an incident, the response officer in charge of a clean-up operation will have to give consideration to protection priorities to be employed during the response.  They are, in order of preference, the following:

a)	Human health and safety;

b)	Rare, threatened and near-threatened fauna and flora;

c)	Habitat and cultural resources;

d)	Commercial resources;

e)	Amenities.

However, in assessing protection priorities, it is necessary to maintain a balanced view of the potential successes of the particular response strategies.  This is of particular importance when it is unlikely that such strategies will be successful in protecting a higher sensitive resource, but could be successful in the protection of less sensitive resources.

4.4	Response Structure

The DEA has a responsibility of setting up a response team and to ensure that the whole  operation is running smoothly.  This will be done in consultation with SAMSA and more specifically the JRC.  Depending on the scale of the incident, the DEA response unit shall comprise of:

a)	On-scene coordinator;

b)	Shore controller;

c)	Logistics officer;

d)	Environmental liaison officer;

e)	Communications officer.

In addition, where shoreline protection and clean-up measures are implemented, a shore logistics officer from the relevant local authority(s) will be appointed to liaise with the DEA response team.

4.5	National Combating Strategies

4.5.1	Surveillance

If coastal or marine resources are not threatened or likely to be threatened, it may be best to do nothing other than continue to monitor the slick and its movements.  If the oil is diesel or light crude, and it is far offshore, the majority of it will evaporate and disperse within 24 to 48 hours, and it will therefore have minimal impact.

A fixed wing maritime surveillance aircraft shall be contracted to the checking and monitoring of the movement of the slick during an incident.   Furthermore, pilots of the surveillance aircraft are required to report any sight of any substance in the sea for immediate onward transmission to DEA or SAMSA and to monitor the movement and behaviour of the spill.  If the assessment shows that another State is likely to be threatened, SAMSA will, through the Department of Foreign Affairs, inform the authorities in the threatened State.

4.5.2	Containment and recovery

Oceanographic conditions off the South African coastline are not generally conducive to the containment and recovery of oil at sea.  Under exceptional circumstances though, this strategy can be attempted in some coastal bays.  Experience with spills in other parts of the world has shown that in the majority of cases it is unlikely that more than 20% of the spilled oil can be recovered at sea. Booms and skimmers are utilised in sheltered ports and coastal areas in an attempt to protect sensitive areas using vessels with a weir skimmer incorporated into its structure.

Attempts should be made to prevent oil reaching the landing beaches of African Penguins and the rocks on which cormorants land because previous experience has shown that when oil reaches such localities very large numbers of birds, or a large proportion of birds in a colony, become oiled.

4.5.3	Dispersion

a)	Mechanical dispersion

For  relatively  small  spills  resulting  in  sheens,  the  speed  of  dispersion  can  be  increased  by  steaming through the slick, where turbulence created by the passage of the vessel will break up the oil. This can also be improved by dragging devices known as breaker boards, or nets behind the vessel. These further agitate the surface of the water and break up the slick. Sometimes the natural dispersion by the waves could be a solution.

- Chemical dispersion

The most common method of dispersion is achieved using chemical dispersants. Oil spill dispersants have frequently been used during response operations.  However, certain dispersants are only effective on certain types of oil and only within a limited time span after the spill.  Recent oil spill dispersant formulations have been designed to treat more viscous and weathered oils which could not be effectively treated by the earlier formulations.

Dispersant application may have both advantages and disadvantages.  Disadvantages include the fact that application of the dispersants result in increased concentrations of oil in the water column, and oil dispersant mixtures are generally more toxic than the oil itself.  It is very important  therefore  that  their use is properly controlled; that they are used only when physical containment and removal is not possible, and that their use results in a net environmental benefit.

South Africa’s national oil spill dispersant policy sets out the criteria necessary for making rational decisions on the non-use of dispersants.  The policy can be found on the SAMSA website in Marine Notice No. 14 of 1997 under the title: “Policy on the use of oil spill dispersants in South African waters” and any  amendment  made  thereafter.

The decision to use dispersants should only be taken by the government authority responsible for the co-ordination of oil spill combating i.e. DEA.  In short the policy on dispersant usage requires and prohibits their use in water depths of less than 30 metres and within five (5) nautical miles of the coast unless a net environmental benefit analysis suggests otherwise.

The patrol vessels of DEA and the vessels that work the SPM (Single Point Mooring) and the CBM (Conventional Buoy Mooring) are dedicated to carrying dispersant.  Additional dispersant stocks are located in the ports of Cape Town and Durban (including 20 tons of Type III concentrate dispersant in Cape Town, formulated to disperse heavy emulsified oils).

Type I dispersants should not be used on emulsified or other viscous oils as they are not effective on these types of oils.

A dedicated aerial dispersant spraying resource is not available in South Africa.  Aerial dispersing could be provided through the use of crop spraying aircraft which are available locally.

Note: Naturelink at Wonderboom airport in Pretoria provides aircraft with dispersant equipment capable of carrying 2 tons of dispersant with a 200 nautical mile range under contract to OSRL in  West and Central Africa.

4.5.4	Shoreline protection and clean-up

a)	Coastal protection

Sometimes clean-up efforts contribute to environmental damage and for that reason it is often better to leave it to natural clean-up processes.  In situations where some clean-up must be attempted, it is important that methods selected are those which are the least damaging to the environment.  Decisions about clean-up will depend on the type of oil, degree of contamination and the type of habitat affected.  Specific guidelines for the protection and clean-up of coastal and marine resource areas are clearly outlined in the DEA local coastal contingency plans.

Booms and barriers can be used to protect coastal features such as estuaries, marinas, harbours and water intakes either by closing off entrances to these features or if this is not feasible, by deploying them so as to deflect or  divert the oil away  from the area of concern. In the latter case the oil should be directed towards a sacrificial area where it can be collected and recovered.  These sacrificial areas are identified in the local coastal contingency plans.

In cases where the oil has already come ashore, booms may be used to trap the oil for collection and to protect adjacent areas from becoming contaminated.  In general though, it is not feasible to successfully place booms along straight sections of beach.  When floating booms are used to close off certain areas, they can be backed up by absorbent booms to pick up any oil, which might penetrate the barrier.  Bubble barriers are popular for the protection of yacht basins and marinas as they do not inhibit the movement of boats in and out of the facility.

Booms or barriers can also be improvised using “materials of opportunity” such as wood, bamboo, fishing  nets and natural absorbents such as straw bales, coconut husks, reeds and sugar cane.  In some cases bulldozers can also be used to construct sand bars across estuaries or lagoons to prevent oil from entering them.  In such cases it may be necessary to insert pipes into the barrier to prevent a build-up of water in the lagoon or estuary.

b)	Coastal clean-up

If due to weather and sea conditions, response at sea is not feasible or protection of sensitive areas is not possible, or these have already been affected, appropriate clean-up priorities and other response measures should be determined. Priorities for clean-up methods will depend on a number of factors including the following:

a)	The use by wildlife of beaches as access to breeding/roosting localities;

b)	The ecological sensitivity of the area concerned;

c)	The importance of the beach for recreational purposes;

d)	The amenability of the beach to natural cleansing;

e)	The degree of oiling;

f)	The accessibility of the beach; and

g)	Seasonal considerations.

Depending on the season, priority will normally be given to heavily contaminated beaches that are either of ecological significance or of high recreational value giving special consideration to the risk of substantial, perhaps irreversible or reversible only in the long term, damage to wildlife populations.  The timing of the commencement of the clean-up operation also depends on a number of factors. In the early stages heavy concentrations of stranded oil which may be refloated only to contaminate other areas, should be cleaned up as soon as possible.

This is also true if the oil is likely to become buried on the beach as a result of cyclical movements of sand on to and off the beach.

On the other hand, if it is not likely to refloat or be buried, then it may be preferable to wait until all the oil has come ashore to avoid having to clean the same area twice.  This is particularly true for contaminated rocks.

Manual clean-up methods are generally preferred for sandy beaches, especially in environmentally sensitive areas although where there are huge accumulations of oil and oily debris it may be necessary to use mechanical equipment such as bulldozers and front-end loaders.

In either case it is important to remove as little as possible of the sand itself both to limit the volume of waste for disposal and to avoid creating erosion problems on the beach.  Clean-up teams need to be closely supervised as much as possible.

4.5.5	Bioremediation

Bioremediation of oiled shorelines in South Africa is considered an optional clean-up strategy for a percentage of areas, and a relatively narrow range of the shoreline.  However, types of substrate beaches are also areas of considerable importance as wildlife habitat, biological diversity, and recreation and subsistence resources.  In these areas bioremediation would be considered a valuable option in the case of a spill to accelerate the natural bioremediation of the oil.  This strategy would also apply to a narrow range of oil conditions and works best on moderately oiled  shorelines.

Even on  beaches of the appropriate substrate, bioremediation would be used as a secondary strategy after mechanical clean-up if the oil was deposited in heavy concentrations.  In addition, natural recovery may be the appropriate strategy on shorelines with only a light coating of oil.

Previous experience has shown that marine vertebrates that breed on land, especially seabirds, are sensitive to oil pollution.  Without intervention, large numbers of these animals will often die after becoming oiled, with severe consequences to their populations.  Where possible, it is necessary to rescue animals that become oiled, for rehabilitation and subsequent return to the wild.  Because rehabilitation, which requires intensive capacity to  provide care, is often limited, it may be  necessary to restrict or locate animals to prevent them from becoming oiled.  The removal of breeding animals from their breeding areas for rehabilitation or relocation may separate them from their young, therefore it may be necessary to collect such young for captive rearing.

Biodegradation is the microbial conversion of hydrocarbons in oil to carbon dioxide and water.  It typically occurs at an oil-water interface and may be limited by the availability of oxygen, moisture, and nutrients, particularly nitrogen and phosphorus.  Biodegradation rate is a function of temperature.

Bioremediation is acceleration of the microbial conversion of hydrocarbons in oil to carbon dioxide and water. This typically occurs through application of constituents where organisms are metabolically limited. On many shorelines, the limiting constituents will be nitrogen and phosphorus, which can be applied in the form of fertilizer. In temperate climates, bioremediation generally works best during summer months, provided sufficient moisture is present.  Low ambient temperatures do not necessarily exclude bioremediation. Activity has been recorded at temperatures as low as -2ºC and biodegradation does occur in arctic areas during summer.

In most cases, other forms of treatment may be required prior to nutrient addition to achieve the desired enhancement rate.  Most areas of the world have indigenous bacteria capable of degrading oil.  The use of products containing non-native bacteria and enzymes is currently not recommended because they are generally not required and may be detrimental.  The decision to use bioremediation should be based on the nature of an oil, the character of an affected shoreline, and the relevant political jurisdiction.  Bioremediation is typically used as a final treatment step after completing conventional shoreline treatment or in locations where other forms of treatment are either not possible or not recommended.  In cases where surface oiling is either moderate or high, water or chemical washing of a shoreline is recommended as a first step to remove bulk oil.

For light surface oiling and subsurface oil, bioremediation may be considered without pre-treatment.  Any pooled oil, mousse, or tar balls should be manually or mechanically removed before attempting bioremediation.  The challenge for bioremediation is to supply nutrients in such a way that they are not applied in excessive amounts and are not washed away by tides, beach runoff, or any water supplied by mechanical sprinkler systems  For similar reasons, application of nutrients to spills on open water is not recommended.

4.5.6	Rocky shoreline clean-up

Oil washed up on to rocky shorelines will collect in rock pools from where it can be removed by using pumps or vacuum trucks.  If the areas are inaccessible by road, the oil can be removed by mixing it with absorbent materials, which can then be removed manually using forks or rakes to take it to a collection point.  Once the liquid oil has been removed, it may be necessary to clean those rocks which have been left with a surface film of oil. This is generally true for heavily utilised amenity areas, although in more remote areas the oil could be left to weather naturally.

Cleaning up can be undertaken using high-pressure hoses with cold or hot water depending on the nature of the oil. In the case of very viscous oil it may even be necessary to use steam cleaning and to apply detergents or dispersants to the rocks prior to cleaning.

In extreme cases where oil has been baked onto the rocks, rocks may have to be sand-blasted.  Whichever method is selected, provisions should be made to collect the oil that is removed either using booms and pumps or absorbent materials.

In selecting the clean-up method, consideration must be given to the fact that the more aggressive the method, the less likely that any of the marine organisms on the rocks will survive the treatment.  Their use should therefore be restricted to amenities.

As indicated above, the activities which are to be undertaken in the event of an oil spill impacting or threatening to impact any particular section of the coastline are outlined in the DEA local coastal contingency plans.

4.6	Oil Combating Equipment

The national oil pollution combating equipment is under the maintenance and care of  DEA. Arrangements can be made through the DEA representatives for the lease of the equipment.  The oil industry and port authorities have stocks of oil spill equipment at various locations around the coast.

In contrast to the above, the equipment and materials used in beach clean-up operations are, for the most part, not specifically for oil spills.  They are provided by local authorities themselves or are available from private contractors.

Equipment type and uses

| Equipment | Use |
| --- | --- |
| Boom | Containing oil for recovery |
| Anchors | Anchoring boom |
| Floats | Marker and trip-line buoys |
| Rope | Towing, anchoring |
| Paravanes | High-speed towing |
| Chain | Anchoring |
| Cable | Towing, positioning boom |
| Lights | Night-time illumination of boom |
| Fittings | Connecting boom, lights, anchor lines |
| Power supply/generator | Provide power for air blowers, pumps, etc. |
| Air blowers | Filling air chambers on inflatable boom |
| Pumps | Filling water chambers on shore-sealing boom |
| Vessels | Deploying, towing, and tendering boom |
| Aircraft | Slick spotting, positioning of vessels |
| Radios | Communication among vessels/aircraft |
| GPS | Positioning boom, documenting activities |

4.7	Disposal of Pollutants and Debris

Clean-up operations can generate substantial quantities of oily debris.  Temporary storage, transportation and final disposal methods shall be arranged to comply with the DEA disposal  approvals.  This will usually be facilitated by the responsible local authority.  In the case of an oil spill, the DEA coastal oil spill contingency plans contain information on the disposal of oily debris . This should include any pre-designated arrangements for disposal sites and approved contractors.  Ideally, disposal sites should be identified as close as possible to those areas where oil pollution could most likely occur.

Each type of waste has a different optimal disposal method.

It is important to:

  - Segregate wastes by type.

  - Minimize the quantity of each type.

  - Avoid mixing hazardous and non-hazardous wastes together to prevent creating a larger volume of hazardous waste.

  - Label all waste containers and identify the source.

Non-oily wastes (e.g., sewage, domestic waste) that are generated during clean-up operations can be disposed of at local wastewater treatment plants and municipal landfills.  Options for disposing of oiled and hazardous wastes (subject to regulatory requirements) include:

- Industrial landfilling

- Land farming

- Open burning

- Portable incineration

- Commercial incineration

- Reprocessing

- Reclaiming/recycling

Local plans contain information on waste management and disposal in their zones.

PART V: ANCILLARY ACTIVITIES

5	Media Response

5.1	Press conferences

As the major pollution incident is usually of immediate interest to both local and international media, it is in the public’s interest to keep the media fully informed and as regularly as possible.  Failure to do so at an early stage could result in seriously distorted information circulating both nationally and internationally.  The media should receive reliable information timeously.

It is the responsibility of SAMSA to make information available as soon as possible.  It is good practice to ensure that the JRC has a media office in place.  A media team and designated public relations officers should take control of the media office and liaise with relevant Ministries and the press as soon as required.  They should arrange press conferences when instructed to do so by the JRC.  No journalists should have direct contact with the response team during the operation as this can disturb the urgent processes of the operation.

5.2	Ministerial and VIP visits

It is usual that when a major incident occurs, the Ministers concerned wish to visit the scene.  He/she is usually accompanied by Senior Officers and other VIPs.  The public relations officers from such Ministries shall make the necessary arrangements for such visits and also try by all means to accommodate journalists when requested to do so on condition that the Minister agrees.

6	Record Keeping and Management

SAMSA is responsible for securing evidence for possible use in court if SAMSA’s principal officer or its Operations Manager has reason to believe that an offence has been committed and there is no admission of contravention on the part of the polluter.  The gathering of such evidence must not interfere with operational activities of the salvors and other emergency services.

7	Reporting

Each government department or agency responsible for the prevention and combating of pollution shall on the termination of a response compile a report detailing their actions related to the incident and the results achieved.  Lessons learnt shall be documented and be made available to all role players for training purposes and in preparation of any future incidents.

8	Training and Exercises

SAMSA and DEA are to conduct regular training programmes and exercises for stakeholders and role players.  The training programmes are designed to enable South Africa to have sufficient numbers of trained personnel to mount a credible and effective response to a pollution incident.  Training programmes shall be conducted on three levels, and which recognise the overall technical complexities of managing a spill response and that the associated  knowledge  required  by  personnel  varies  depending  on  their  levels  of responsibility.

The following shall be the three levels of training to be conducted:

(1)	Senior Management:  Level 3:  The focus of training shall be on the requirements of members of the national authorities from DEA and SAMSA responsible for high level decision making.

(2)	Middle management: Level 2:  The focus shall be on the requirements of middle management personnel including designated shore logistics officers and area controllers responsible for the preparation of contingency and response plans and the management and conduct of effective spill response operations and associated logistics, administrative and financial tasks.

(3)	Operator: Level 1: The focus shall be on the requirements of operational personnel, those undertaking on-site clean-up operations and operating spill response equipment.

The cost of Level 3 training shall remain with SAMSA while that of Levels 1 and 2 will be the responsibility of DEA.  It must be emphasised that offshore installations, terminals and industrial stakeholders must ensure adequate investment in the training and exercising of their personnel.

Exercises should validate the effectiveness of the plan and the readiness of stakeholders and role players who should be part of these initiatives so they can learn their roles, develop relationships and improve the plan where necessary.  The equipment should be tested to ensure that it is in good working condition.

Lessons learnt should be noted and improvements in the plan should follow.  For the purposes of sharing responsibility DEA and SAMSA should share the responsibility and cost of an exercise as this could be very expensive.  One should note that a real-time scenario is envisaged as part of the exercise.

9	Maintenance and Review of the Plan

Contingency plans are evolving documents and as such require regular updating.  The Plan shall be subject to a bi-annual review taking into account policy changes and experience gained from incidents and exercises.  The JRC should  assess  the  relevance,  adequacy,  and effectiveness of the Plan after each incident.

Should there be corrective or improvement areas identified, they should be reviewed immediately and communicated to all stakeholders and role players.  Amendments will be handled by a SAMSA/DEA liaison committee according to suggestions  by  concerned  and  affected  parties.  An urgent need to review certain parts immediately should only be done in the interest of improving effectiveness.

Information for updating the Plan should be forwarded on a regular basis to the Executive Head: Centre for Sea Watch and Response (CSWR).

Appendixes

Appendix 1

Local Coastal Contingency Plans

The local coastal contingency plans cover the following:

a)	Legal aspect of the oil spill response including liabilities, claims and costs;

b)	Reporting procedures and activation of the Plan;

c)	Organisational structures during an incident and job descriptions of role players;

d)	Available facilities and resources;

e)	Geographical boundaries of responsibility;

f)	Site-specific protection and clean-up measures required during an incident;

g)	List of available equipment; and

h)	Contact details of all role players.

The plans contain the following information which could be of assistance during a major incident:

a)	Guidance on what equipment and personnel would be at the disposal of the national response unit  when needed;

b)	Arrangements for establishing accommodation and catering for members of the response team who      would be usually away from their base;

c)	Arrangements for communication purposes;

d)	Arrangements for accommodating press conferences;

e)	Temporary, intermediate and final storage site and routes for disposal of waste; and

g)	Maps clearly showing access points, terrain types, etc.

Appendix 2

Telephone List for Oil Spill Reporting

The Principal Officer (PO) stationed closest to the incident should be the first contact

| Organisation/Location |  |  | Office | Fax | After hours | Cell |
| --- | --- | --- | --- | --- | --- | --- |
| SAMSA |  |  |  |  |  |  |
| SAMSA Eastern Region [KZN] |  |  |  |  |  |  |
| Capt. Saroor Ali (Regional Manager ) | (031) 307 3006 |  |  | (031) 307 3086 | (031) 2617919 | 071 686 9593 |
| Durban office |  |  |  |  |  |  |
| Mr G. Conway (Principal Officer ) | (031) 307 1501 |  |  | (031) 306 4983 | 082 449 6350 | 082 449 6350 |
| Richards Bay office |  |  |  |  |  |  |
| Capt. Daron Burgess (Principal Officer) | (035) 788 0068 |  |  | (035) 788 0067 | 082 374 7942 | 082 374 7942 |
| SAMSA Southern Region [Eastern Cape] |  |  |  |  |  |  |
| Capt. Nigel Campbell (Regional Manager) | (041) 582 2138 |  |  | (041) 582 2130 | (041) 368 7528 | 083 309 6053 |
| Port Elizabeth office |  |  |  |  |  |  |
| Mr Brian Colenutt (Principal Officer) | (041) 585 0051 |  |  | (041) 582 1213 | (041) 360 3744 | 082 445 3167 |
| Coega office |  |  |  |  |  |  |
| Mr Brian Colenutt |  | (041 ) 585 0051 |  | (041) 582 1213 | (041) 360 3744 | 082 445 3167 |
| East London office |  |  |  |  |  |  |
| Capt. Peter Kroon (Principal Officer) | (043) 722 4120 |  |  | (043) 722 2264 | (043) 726 5857 | 082 445 3166 |
| Mossel Bay office |  |  |  |  |  |  |
| Mr. Dave Manley | (044) 690 4201 |  |  | (044) 691 1206 | (044) 695 1844 | 082 477 1813 |
| SAMSA Western Region [Western Cape] |  |  |  |  |  |  |
| Capt. Dave Colly (Regional Manager) | (021) 421 6170 |  |  | (021) 419 0730 | 083 412 8861 | 083 412 8861 |
| Cape Town office |  |  |  |  |  |  |
| Capt. Gustav Louw (Principal Officer) | (021) 421 6170 |  |  | (021) 419 0730 | 071 608 8579 | 071 608 8579 |
| Saldanha Bay office |  |  |  |  |  |  |
| Mr. Martin Slabber (Principal Officer) | (022) 714 1612 |  |  | (022) 714 3635 | (022) 714 1427 | 082 789 6764 |
| Port Nolloth office |  |  |  |  |  |  |
| Mr. Justin Coraizen | (027) 851 7695 |  |  | (027) 851 7699 | 082 386 2141 | 082 386 2141 |

| Organisation/Location |  |  | Office | Fax |  | After hours |  | Cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Centre for Sea Watch And Response (CSWR) |  |  |  |  |  |  |  |  |
| Plattekloof |  |  |  |  |  |  |  |  |
| Mos. Nontsindiso Kunene | (021) 938 3316 |  |  | (021) 938 3319 |  | 082 922 8395 |  | 082 922 8395 |
| Capt. Ravi Naicker | (021) 938 3308 |  |  | (021) 938 3319 |  | (021) 529 0415 |  | 082 768 8401 |
| MRCC | (021) 938 3310 |  |  | (021) 938 3319 |  |  |  |  |
| Capt. Karl Otto (Exec. Head : CSWR) | (021) 938 3317 |  |  | (021) 938 3334 |  | 082 812 2991 |  | 082 812 2991 |
| SAMSA Head Office |  |  |  |  |  |  |  |  |
| Exchange |  | (012) 366 2600 |  | (012) 366 2601 |  |  |  |  |
| Mr. Sobantu Tilayi (Exec. Head: Shipping) |  | (012) 366 2624 |  | (012) 366 2601 |  | 071 608 6480 |  | 071 608 6480 |
| Mr. T. Mokhele (CEO) |  | (012) 366 2603 |  | (012) 366 2601 |  |  |  |  |
| Department of Environmental Affairs |  |  |  |  |  |  |  |  |
| Dr. Yazeed Peterson (DD) | (021) 819 2450 |  |  | (021) 819 2445 |  | 083 530 3127 |  | 083 530 3127 |
| Ms. Ferosa Albertus Stanley | (021) 819 2457 |  |  | (021) 819 2445 |  | 072 173 6234 |  | 072 173 6234 |
| Mr. Tebogo Ntje | (021) 510 7420 |  |  | (021) 511 2679 |  | 082 557 6612 |  | 082 557 6612 |
| ????? | (021) 510 3957 |  |  |  |  |  |  |  |
| Smit Amandla Marine (Pty.) Ltd. |  |  |  |  |  |  |  |  |
| Capt. Dave Murray | (021) 507 5777 |  |  | (021) 507 5885 |  | (021) 797 2711 |  | 082 909 9948 |
| Ms. Nomkitha Mbele | (021) 507 5777 |  |  | (021) 507 5885 |  | 082 557 6612 |  | 079 699 4406 |
| Capt. Dave Main | (021) 507 5777 |  |  | (021) 507 5885 |  | (021) 794 6662 |  | 082 907 2775 |
| Mr. Paul Maclons | (021) 507 5777 |  |  | (021) 507 5885 |  | 082 909 2013 |  | 082 909 2013 |
| Transnet National Ports Authority |  |  |  |  |  |  |  |  |
| Cape Town VTC |  | (021) 449 1009 |  |  | ???? |  | (021) 449 2039 | ??????? |
| Coega VTC |  | ????? |  |  | ??? |  | ????? | ?????? |
| Durban VTC |  | (031) 308 8262 |  |  | ???? |  | (031) 361 8300 | ??????? |
| East London VTC |  | (043) 700 2142 |  |  | ??????? |  | (043) 700 2143 | ?????? |
| Mossel Bay VTC |  | (044) 604 6287 |  |  | ???? |  | (044) 604 6287 | ????? |
| Port Elizabeth VTC |  | (041) 507 1910 |  |  | ????? |  | (041) 585 4950 | ????? |
| Richards Bay VTC |  | (035) 905 3401 |  |  | ????? |  | (035) 905 3061 | ??? |
| Saldanha Bay VTC |  | (022) 703 5310 |  |  | ????? |  | (022) 703 4116 | ????? |

Appendix 3

Procedure for Responding to Chemical or HNS Spills

| Who | Step | Activity |
| --- | --- | --- |
| Reporter | 1 | Report initial incident to CSWR or SAMSA Port Office |
| CSWR or Principal Officer (PO) | 2 | Collects all possible information and completes initial report as per Appendix 1 |
| CSWR or PO | 3 | Informs Executive Head :CSWR Informs Executive Head : Shipping |
| Executive Head: CSWR | 4 | Mobilise SAMSA Casualty Response Unit (CRU) to investigate, assess and report |
| CRU | 5 | Investigate, assess and report to GM: Ops Report as per Appendix 2 |
| Executive Head: CSWR | 6 | Consult EA and makes decision: A. No further action B. Implement response and form Joint Response Committee (JRC) |
| JRC | 7A | Decides on response (informs and consults with specialized agencies) |
|  | 7B | Execute response |
|  | 7C | Arrange for disposal or storage of contaminated wastes recovered (in compliance with legal requirements) |
| Executive Head : CSWR Executive Head : Shipping | 8 | Terminates action on satisfactory completion of JRC response. |
| Executive Head :CSWR Executive Head : Shipping | 9 | Initiates de-briefing with role-players |

Appendix 4

Procedure for setting up a Joint Response Committee (JRC)

| Who | Step | Activity |
| --- | --- | --- |
| Executive Head: CSWR | 1A | Consults with stake-holders and role-players and constitutes a Joint Response Committee (JRC) |
|  | 1B | Executive Head: CSWR shall be the chair, or Executive Head: CSWR shall appoint in writing a SAMSA official (Senior Surveyor) to be chair |
| Chair: JRC | 2 | Instructs the Casualty Response Unit (CRU) to set up a Joint Operations Centre (JOC) with an appropriate communication system and at a convenient location |
| JRC | 3A | JRC shall meet regularly (as often as is required) |
|  | 3B | JRC shall discuss, inform, formulate and agree on plans to prevent and combat the pollution (in a transparent manner) and instruct the CRU and other role-players accordingly |
|  | 3C | JRC shall conduct its business as per Annex 5 (The JRC Management and Media Protocols) |
| Chair: JRC | 4 | After consultation with committee may terminate the response (in writing) |

Appendix 5

JRC Management Protocol & Media Response Protocol

| A | JRC Management Protocol |
| --- | --- |
| 1 | Members of the JRC shall be appointed by the General Manager Operations, assisted by the DEA, on the basis of any of the following: - extensive knowledge of matters relating to management of response operations in a pollution incident; - expertise in marine salvage matters; - expertise in ship construction; - extensive knowledge of the spilled substance, including its behaviour in the environment; - knowledge of law relating to any aspect of the incident; - shipowner, terminal operator and / or agent; - representative of the defence force, police services, wildlife organisation/s; - representatives of the major stakeholders (SAMSA and the DEA); - representative of a role player having a responsibility for some part in the prevention or combating operation and who could or will incur costs - representative of TNPA - representative of the local authority - representative of Provincial departments of; SANDF, SAN Parks, SA Museum, Prince Edward Island management committee - representative of any other body considered to have specialised knowledge or expertise with regard to the incident at hand |
| 2 | The JRC shall conduct its business in an open manner with the overall aim of mitigating the effects of the incident. |
| 3 | The JRC shall decide on what preventive and / or combating measures to be taken in order to mitigate the impact of the pollution incident on the environment, keeping in mind the priorities set out in the Plan and the balance of funds available in the budget. |
| 4 | The JRC is responsible for making a request for external assistance, should it become necessary. |
| 5 | The JRC shall meet regularly, usually daily, sometimes twice a day depending on the incident. As the urgency of the incident decreases, the frequency of the JRC meetings may be reduced. |
| 6 | The JRC approves the independent auditor nominated by SAMSA, whose responsibility is to keep records of the operation and costs involved. |
| 7 | The JRC shall provide a coordinated and factual response to the media, for SAMSA to use in briefing the press. |
| 8 | The JRC shall advise the Chairman to order termination of response after it is satisfied that the operation has been effectively completed. |

| B | Media Response Protocol |
| --- | --- |
| 1 | A media office shall be established, as part of the JOC, by the CRU. |
| 2 | The JRC, led by the Chairman, shall appoint a media team, and designated public relations officers to take control of the media office and liaise with the relevant ministries and the press. |
| 3 | The media team shall arrange press conferences when instructed to do so by the JRC. |
| 4 | SAMSA has the responsibility to brief the press. Typically, the contents of the briefing will be a coordinated and factual response compiled by the JRC. |
| 5 | When need arises, specific stakeholders and role-players will be (with their consent) briefed by the JRC to meet with the press in a media conference. |
| 6 | Should the Minister/s and / or VIP’s from the relevant ministries wish to visit the incident scene, the public relations officers from those ministries shall liaise with the designated public relations officers at the incident scene to make the necessary arrangements for such visits. |
| 7 | The public relations officers from the ministries shall try to accommodate journalists when requested to do so, on condition that the Minister agrees. |
| 8 | Journalists shall not be allowed to have direct contact with the response team during the operation, as this can disturb the urgent processes of the operation |

Appendix 6

# DEA Mobilisation Procedure

Figure 1: The following schematic diagram shows the Equipment Mobilization Procedure (EMP) in response to and oil spill NB: Please note that this procedure may vary (from incident to incident) based on the assessment report from the incident site

Appendix 7A

Procedure for integrating lessons learnt into the Plan

| Who | Step | Activity |
| --- | --- | --- |
| Executive Head: CSWR | 1 | In consultation with DEA appoints coordinator(s) and evaluator(s) |
| Coordinator(s) | 2A | Set key objectives of exercise (communications, equipment and/or response readiness) |
|  | 2B | Determine scope of exercise (where, how long and/or who to involve) |
|  | 3A | Control exercise (monitor, control and facilitate) |
|  | 3B | Collect data |
|  | 3C | Document findings (report) |
| Evaluator(s) | 4A | Observe response and compare with objectives |
|  | 4B | Measure performance |
|  | 4C | Document findings (report to coordinator) |
| Coordinator(s) | 5 | Collect data from all participants and role-players, analyse data and report findings and recommendations to GM: Ops |
| Executive Head: CSWR | 6A | Reviews report and, where appropriate, consults with SAMSA/DEA liaison committee on amendment(s) to the Plan |
|  | 6B | Submit the proposed amendment(s) to the bi-annual review workshop for formal adoption |
| Executive Head: CSWR | 7 | Amends the Plan and disseminates these amendment(s) |

Appendix 7B

Procedure for integrating lessons learnt into the plan and amendment thereof

| Who | Step | Activity |
| --- | --- | --- |
| Executive Head: CSWR | 1 | Deal with incident as prescribed in this Plan |
| Executive Head: CSWR | 2A | Collect data, reports and other information from all role- players |
|  | 2B | Convene a de-briefing session with all role-players and analyse all relevant data. |
|  | 2C | Document findings (report) |
| Executive Head: CSWR | 3A | Reviews report and, where appropriate, consults with SAMSA/DEA liaison committee on amendment(s) to the Plan |
|  | 3B | Submit the proposed amendment(s) to the bi-annual review workshop for formal adoption |
| Executive Head: CSWR | 4 | Amends the Plan and disseminates these amendment(s) |

Appendix 8

# SAMSA Initial Response Check List

|  |  | Yes | No |
| --- | --- | --- | --- |
| 1 | Has the initial pollution incident report from an observer been received by the Principal Officer, stating : |  |  |
| 1.1 | Name of person reporting the incident |  |  |
| 1.2 | Telephone number or other means of contact |  |  |
| 1.3 | Date & time of observation |  |  |
| 1.4 | Details of observation |  |  |
| 1.5 | Location ( lat/long or bearing & distance) |  |  |
| 1.6 | Source & cause of pollution |  |  |
| 1.7 | Type & estimated quantity of oil spilled and potential or probability of further pollution |  |  |
| 1.8 | Has the master bee requested to submit the cargo manifest |  |  |
| 1.9 | Weather & sea conditions at the time of observation |  |  |
| 1.10 | Action taken or intended to respond to the incident |  |  |
| 2 | Has the PO informed the Executive Head of the pollution incident |  |  |
| 3 | Has the Executive Head issued instructions to mobilize the CRU |  |  |
| 4 | Have the CRU members |  |  |
| 4.1 | Read the initial pollution incident report |  |  |
| 4.2 | Read the Executive Head instructions issued to the PO |  |  |
| 4.3 | Prepared the appropriate protective clothing |  |  |
| 5 | Have the CRU members obtained &arranged : |  |  |
| 5.1 | Hiring appropriate mode of transport to the pollution incident scene |  |  |
| 5.2 | Air travel bookings to the pollution incident scene and accommodation |  |  |
| 5.3 | Any necessary expenses during the extent of the operation |  |  |
| 6 | At the pollution incident scene the CRU is ,among other things ,to carry out the following actions |  |  |
| 6.1 | Determine the name and address of the vessels owner |  |  |
| 6.2 | Inquire into the insurance status of the vessel and the name of the P & I Club |  |  |
| 6.3 | Obtain the cargo manifest of the ship |  |  |
| 6.4 | Request the details of the fuel oils, and other harmful substances on board HFO ,DO, LO |  |  |
| 6.5 | Evaluate the specific threat of the pollution posed by the vessel in respect of bunkers and/or cargo on board |  |  |
| 6.6 | Obtain any additional particulars ,calculations or considerations required by the Executive Head |  |  |
| 6.7 | Convene JRC on instructions from the Executive Head and establish a JRC |  |  |
| 6.8 | Prepare press releases |  |  |
| 6.9 | Inform affected authorities and parties |  |  |

Appendix 9

# DEA Initial Response Check List

| 1 | Spill Data |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. | Name of incident : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B. | Date and Time of Incident: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C. | Type of Incident: |  | Grounding |  | Explosion |  | Collision |  |  | Transfer Operations |  |  | Other |  |
| D. | Spill Location: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E. | Distance to nearest land: |  | (Nm or Km) |  |  |  |  |  |  |  |  |  |  |  |
| F. | Did Source Burn: |  | Yes: |  |  |  |  |  | No: |  |  |  |  |  |
| G. | Spill location: |  | Longitude: |  |  |  |  |  | Latitude: |  |  |  |  |  |
| H. | Estimated volume of product released: |  |  |  |  | (Lt or Tonnes) |  |  |  |  |  |  |  |  |
| I. | Release Status: |  |  | Stopped |  | Continuous |  |  | Unknown |  |  |  |  |  |
| J. | Is Spill: | Contained? |  | Spreading? |  | Unknown? |  |  |  |  |  |  |  |  |
| K. | If continuous, average rate of product being released: |  |  |  |  |  |  |  | (Lt or Tonnes/ Hour) |  |  |  |  |  |
| 2. | Weather & Water Conditions (At time & location of the spill) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A. | Temperature: |  |  | Air: |  | °C |  | Water: |  |  | °C |  |  |  |
| B. | Weather: | Clear |  | Partly Cloudy |  |  |  | Overcast |  |  | Rain | Snow |  | Fog |
| C. | Tidal State: | Slack Tide |  | Incoming |  | Outgoing |  |  |  |  |  |  |  |  |
| D. | Wind Speed: |  |  |  |  | (knots) |  | Direction: |  |  |  |  |  |  |
| E. | Sea State: | Calm |  | Choppy |  | Swell |  |  |  |  |  |  |  |  |
|  | Waves: | <1m |  | 1-3m |  | >3m |  |  |  |  |  |  |  |  |
| F. | Water Depth: | 0-3m |  | 3-5m |  | 5-10m |  | 10-30m |  |  | >30m |  |  |  |

| 3. | Environmental Data |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. | Biological Resources at Risk: |  |  |  |  |  |  |  |  |  |  |
|  | Habitats: | Intertidal Areas | Estuaries | Coral Reefs | Beaches |  |  |  |  |  |  |
|  | Organisms: |  |  |  |  |  |  |  |  |  |  |
|  | Species | Status (Indicate With A Tick) |  | Distribution |  | Estimated Number Of Individuals |  |  |  |  |  |
|  | Name | Threatened | Not threatened | General | Concentrate |  | 1-10 |  | 11-50 | 51-100 | >100 |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |
| B. | Human Resources at Risk | Commercial facilities and Enterprises |  |  | Public facilities and Enterprises |  |  |  |  |  |  |

Appendix 10

Limitation of Liability

As highlighted in section 2.4 of the plan, the owner has the right to limit his Liability for oil pollution damage. (The owner has a right to Limit its Liability under other circumstances which will not be discussed here).

Consequently there are limited funds to support costs incurred while dealing with preventing and combating of pollution.

Section 09 (5)(b) of the Marine Pollution (Control and Civil Liability) Act 6 of 1981 defines  the actual limits of liability :

“(5)	If the owner of any ship, tanker or offshore installation incurs a liability in terms of the provisions of subsection (9)(1) for any loss or damage suffered or costs incurred as a result of an incident which occurred without such owner's actual fault or privity —

- the provisions of section 261 of the Merchant Shipping Act 57 of 1951 shall not apply in respect of such liability;

- the aggregate of all amounts payable by such owner in respect of such liability, in so far as it relates to a particular incident, shall not exceed—

- in the case of a ship or a tanker, one hundred and thirty-three units of account for each ton of the ship's or tanker's tonnage, or fourteen million units of account, whichever is the lesser;

ii)	in the case of an offshore installation, a sum determined by the Minister, but not exceeding fourteen million units of account.”

“ "unit of account" means a Special Drawing Right as defined by the International Monetary Fund, and the value of such Special Drawing Right in South African currency shall be calculated in accordance with the method of valuation applied by the International Monetary Fund and which is in effect at the time when payment is made, or, in the event of an application in terms of section 12(1), at the time when such application is considered by the court;

(b)	the tonnage of a ship or a tanker shall be its net tonnage with the addition of any engine room space deducted for the purpose of ascertaining its net tonnage.”

1 SDR = R 11.10 (as at 30 August 2007)

However for tankers the State has recourse to the Fund (CLC Protocol 1992) should the Liability limit of Act 6 of 1981 be exceeded.

Article V of the 1992 CLC Protocol explains the Limits of Liability in respect of any one incident:

##### “1.	The owner of a ship shall be entitled to limit his liability under this Convention in respect of any one incident to an aggregate amount calculated as follows:

###### (a)	3 million units of account  for a ship not exceeding 5,000 units of tonnage;

###### (b)	for a ship with a tonnage in excess thereof, for each additional unit of tonnage, 420 units of account in addition to the amount mentioned in subparagraph (a); provided, however, that this aggregate amount shall not in any event exceed 59.7 million units of account “.

For the purpose of this Article the ship's tonnage shall be the gross tonnage calculated in accordance with the tonnage measurement regulations contained in Annex I of the International Convention on Tonnage Measurement of Ships, 1969.

The Fund can be constituted by depositing the sum equivalent to the Limit of Liability, producing a bank guarantee or any other guarantee that may be acceptable to SAMSA or  a Court.

Appendix 11

# Initial Hazardous Noxious Substance Spill Report

Where does this fit in, and what should it be called

| General Information |  |
| --- | --- |
| Date |  |
| Time of spill |  |
| Time reported |  |
| To whom reported |  |
| Position/location of spill |  |
| Description of incident |  |
| VESSEL INFO |  |
| Vessel name |  |
| Call sign |  |
| Vessel type |  |
| Year built |  |
| IMO no. |  |
| Gross tonnage |  |
| Nett tonnage |  |
| Owners name |  |
| Owners agent |  |
| P & I Club |  |
| Crew list |  |
| HNS INFO |  |
| Product name |  |
| UN No. |  |
| Pollution category |  |
| Tank/container details |  |
| Action |  |
| Other Information |  |
| Master /crew action |  |
| Masters report |  |
| Weather |  |
| Vessel draft |  |
| Stability booklets available |  |
| P & I club details |  |

| Certificate of fitness |  |
| --- | --- |
| Date issued |  |
| Date expires |  |
| Last annual survey |  |
| By whom |  |
| Ship /shore checklists |  |
| Cargo loading plan |  |
| Deposit |  |
| Receipt number |  |
| Detention |  |
| Release |  |
| Principal Officer comments |  |

Appendix 12

# This report is for use when reporting discharge or potential discharge of noxious liquid substance carried in bulk

| A | Ship name ,call sign/ship station identity and flag |  |
| --- | --- | --- |
| B | Date & time of events ( LT) |  |
| C | Position : latitude & longitude |  |
| D | Position : True bearing & distance |  |
| E | True course |  |
| F | Vessel speed |  |
| L | Route information |  |
| M | Radio communications including INMARSAT |  |
| N | Time of next report |  |
| P | Technical name of noxious substances on board |  |
|  | UN number |  |
|  | Pollution category (A,B,C or D) for each substance |  |
| Q | Condition of ship |  |
|  | Ability to transfer cargo/ballast/fuel |  |
| R | Technical name of noxious substance discharged into the sea |  |
|  | UN number/s |  |
|  | Pollution category (X,Y,Z and OS) |  |
|  | Names of manufacturers or consignee or consignor |  |
|  | Quantity of spill |  |
|  | Whether substance floated or sank |  |
|  | Whether loss is continuing |  |
|  | Cause of spill |  |
|  | Trajectory of spill |  |
|  | Surface area of spill |  |
| S | Weather conditions |  |
| T | Owners contact details |  |
| U | Ship size & type |  |
| X | Action being taken with regard to the discharge and to the movement of the ship |  |
|  | Assistance of salvage efforts which have been requested or to which have been provided by others |  |

Addendums

Addendum 1

# South Africa Oil Spill Response Equipment

|  | Institution | DEA | OPCSA | TNPA | Contractors | Oil Industry |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Cape Town | 3 x 47m inshore patrol vessels with 5 000 lts dispersant tanks carrying chemserve OSE 970 concentrate and breaker boards 1 x 83m offshore patrol vessel fitted with 50 000. lts dispersant tanks carrying Chemserve OSE 750 Oil dispersant stocks - 59000200 Chemserve OSE 750 - 20000 chemserve OSE 970 One patrol/reconnaissance aircraft (Kuswag 8) Booms - 14 x 25m port boom - 30 x 25m Seaguardian boom - 5 x 25m Shoreguardian boom - 7 x 10m Shoreguardian boom - 1 x 20m Riverboom on reel Skimmers - 2 x T-disc skimmers - 1 x Rotadisc 15 skimmer - 1 x Desmi Ocean skimmer (detachable Desmi DOP 250 Archimedes screw pump) 1 x 25m3 Aristock inflatable floating tank 2 x 10m3 Fastanks 1 x 4m inflatable boat with 2 x 60hp engines 1 x drum-vac unit 1 x karcher high pressure water washer |  | 1 x rope oil skimmer air driven recovery rate 10 000 L/H 1 x rope oil skimmer air driven recovery rate 8 000 L/H 1 x disc oil skimmer, power pack and pump, recover rate 10 000L/H 1 x drum oil skimmer recover rate 5 000 L/H hour designed to be fitted to vessel and powered off vessel hydraulics 1 x Dezmi oil skimmer, power pack mounted in a container on a trailer, recovery 10 000 L/H 30 x oil booms foam filled 35 x oil booms inflatable type 1 x trailer equipped with high pressure water jet and dispersant spray pump 1 x equipment trailer fitted with an electrical generator 1 x workboat with catamaran type hull 9m, 2 x 40hp outboat motors 1 x 5m dinghy mounted on a trailer, 25hp outboard motor 1 x debris and oil recovery vessel water witch design, diesel inboard motor complete with a hopper barge and three skips 1 x electric generator. 1 x mobile air compressor 1 x sand piper pump air powered 1 x hiab crane mounted on quay 2 x spate diesel powered water pumps. 1 x12 ton mobile crane. 1 x 10 ton flatbed truck. 1 x trailer mounted fuel bowser with pump. 2 x 10 000L fast tanks. | Oil Spill Control 100m Riverboom 1 x 750 rope mop skimmer 1 x 9m3 Fastank 2 x 3m3 Rapide Fastank Sorbent materials 24 hour Spill Response 100m Portboom 2 x 800mm floating weir skimmers 5 x 10m3 vacuum trucks, Sorbent materials | 4 x 75m Portboom 2 x 25m Portboom 2 x 25m Beach boom 1 x 750 Rope-mop skimmer 1 x Desmi DOP 250 skimmer 2 x 10m3 Fastanks 1 x Ro-vac unit |
| 2 | Coega |  |  |  |  |  |
| 3 | Durban |  |  | 3 x Boom Trailers 1 x Air compressors 2 x Power packs 3 x Spate Pumps P.P.E 2 x Troil Booms 1 x Vikoma Skimmer Hydraulic Pipes Towing bridles Air Pipes 2 x Fast Tanks 2 x Plastic bags for fast tanks 2 x Garbage Trailers 1 x Disc Skimmer 1 x Chemical Skimmer 1 x Chemical Trailer 1 x Vacuum tanker 1 x Pollution craft - Udoti 1 x Quick Response trailer 2 x Porta packs 2 x Traffic lights 2 x Danger triangles 1 x Foot pump 4 x No smoking signs 5 x Booms Container for spate pump Fire hose 2 x Water waders Hose connections 3 x Skimmers 1 x 5 KVA alternator 2 x Inflatable booms 3 x Absorbent in bags 2 x Loose absorbent fibre in drums 2 x Oil resistant pipes 2 x Flood lights 2 x Safety signs men at work 1 x Oil Recovery Vehicle | Drizit Environmental 3 x rope mop skimmers (750, 1 500 & 3 000) 3 x floating weir skimmers (75mm & 40mm) 1 x Rotabrush 5 skimmer 250m Riverboom 250m Portboom 40m Beach boom 3 x Fastanks (2 x 9m3, 1 x 5m3 & I x 1,5m3) Sorbent materials ABZorbit Hazmat Support Services 2 x 750 rope mop skimmers 5 x 70mm floating weir skimmers 1 x disc skimmer 200m Riverboom 2x 10m3 vacuum tankers 3 x 3m3 Rapide Fastank Sorbent materials | 4 x 75m Portboom 2 x 10m Beachboom 2 x 25m Portboom 1 x 1 Om & 1 x 15m Riverboom 1 x Desmi DOP 250 skimmer 1 x Ro-vac vacuum unit 2 x 10M3 Fastank 1 x ?I dispersant spray pod for aerial use |
| 4 | East London |  |  | 8 x Inflatable booms 1 x Portable water pump 1 x Stainless steel skimmer 2 x Stihl blowers 10 x Hose with camlock couplings 10 x Peat sorb (bags) 1 x Plastic fuel container 1 x Trailer |  | 4 x 75m Portboom 2 x 10m Beachboom 1 x 750 Rope mop skimmer 1 x 10m3 Fastank |
| 5 | Mossel Bay |  |  | 1 x Workboat 1 x Launch 1 x Dinghy with oars & motor 2 x Bakkie 1 x Salvage pump SAPIA Equipment 3 x 25m River booms 1 x Electrical boom inflator 1 x Exhaust inflator 2 x Boom towing bridles 2 x Boom end floats 1 x Boom magnetic coupling 5 x 20m Potapack booms 1 x 750 rope mop skimmer oil skim ropes 1 x Floating pulley 1 x Wilden pump 1 x Set hoses 1 x 7500L fastank 3 x Polythene liners Pipe saddle 1 x Ground mat |  | 3 x 25m Riverboom 5 x 20m Portapack booms 1 x 750 Rope-mop skimmer 1 x 10m3 Fastank |
| 6 | Port Elizabeth |  |  | PORT EQUIPMENT 1 x 500m Portatble booms 5 x Galvanised boom reels 5 x Galvanised boom trailers 3 x Boom fairlead rollers 2 x Sparkproof inflator 2 x Two stroke fuel inflator 1 x High pressure cleaner 1 x Orange airline hose 2 x Couplers female thread & hose type 1 x Fastank with complete tap 1 x Fastank drainage system 50M Black PVC 1 x Aluminium step extension ladder 8 x Drizit Brooms 1 x Red bucket 1 x Black heavy duty bucket 1 x Green polyprop shovel 1 x Flammable cabinet 1 x 20L Diesel Jerry can 1 x 10L Two-stroke Jerry can 2 x Funnels 2 x Scoop net (black) 30 x Heavy duty bags with ties 1 x NPA1 - Rubberduck 1 x DESMI Skimmer/Pump 1 x Powerpack |  | 4 x 75m Portboom 2 x 25m Portboom 2 x 10m Beachboom 1 x 750 Rope-mop skimmer 2 x 10m3 Fastank |
| 7 | Richards Bay |  |  | 5 x 23m Inflatable River Booms 1 x 10m Inflatable River Booms 3 x 3m Fibre Booms 4 x Ships magnets 1 x Boom inflator 2 x 20lt Oil Dispersant 1 x Towing Bridle 1 x Petrol for boom inflator 1 x Diesel Rope Skimmer 1 x Pneumatic skimmer 1 x 9 tons Fast tank and 2 liners Boom securing cables |  | 4 x 75m Portboom 2 x 10m Beachboom 1 x 750 Rope-mop skimmer |
| 8 | Saldanha Bay |  | 1 x 16,5m oil recovery & boom deployment vessel 1 x 13,5m shallow draft work boat 3 x 7m boom maintenance boats 4 x 3,5m work boats 1 x 1 Om oil recovery barge •Booms 1 200m fence boom 300m Vikoma Shoreguardian boom 3 500m Expandi boom 4300 1 500m Expandi boom 7000 Skimmers 2 x OPEC rope-mop units 4 x Komara K12 disc skimmers 1 x Desmi Destroil skimmer 1 x Vikoma Sea Devil disc skimmer 2 x Vikoma Delta floating skimmer heads •4 x Vactank units •2 x 10m3 Fastanks 6 x Aqua tanks 1 x 15 000m3 Tankcontainer 3 x 100m3 flexible floating tanks •3 x Archimedes screw pumps 2 x Helical rotor pumps •1 x Mobile operations & communications centre •2 x High-pressure water washing units •2 x 220V petrol generators & lighting sets | 23 x 4m Enretech Booms, 4-pack 36 x 2m Enretech Booms, 4-pack 20 x 10kg Enretech Cellusorb Absorbents 5 x Sparkpoof spades 5 x Sparkproof brooms 2 x Danger tape 5 x Black bags Degreaser 5 x Gloves 2 x Rakes 4 x Scoop nets 1 x Nylon rope 1 x Sysie workboat 1 x Duiker Skiboat 1 x Rubber duck 1 x 4 man boat 8 x Bakkies 1 x Truck with crane 3 x Generators 1 x Spill response trailer 1 x Backup generator 1 x Recovery pump 3 x Emergency lights 1 x High pressure cleaner |  |  |

| Additional Resources |  |  |
| --- | --- | --- |
| Institution | Location | Equipment |
| SA Navy | Simon’s Town | 2 x 20.5m tugs fitted with 4 0001 dispersant tanks & side arm spray booms. 1 x 20t/hr Ro-clean rope-mop skimmer (Seamop 5060) 1 x 20t/hr Ro-clean disc skimmer 400m Ro-clean roboom 1000 (0.43 x 0.36m) on reels with powerpack |
| Oil Industry | Langlaagte | 2 x 25m Portboom 1 x 1 Om & 1 x 15m Riverboom 2 x 10m3 Fastant 1 x Ro-vac unit |
|  | Bloemfontein | 2 x 25m Portboom 1 x 1 Om & 1 x 15m Riverboom 2 x 10m3 Fastank |
| PetroSA |  | 200m Portboom 150m Riverboom• 2 x disc skimmers 2 x weir skimmers 2 x 1500 rope mop skimmers 3 x 10m3 Fastanks Sorbent materials |

SMIT MARINE SOUTH AFRICA (PTY) LIMITED

Oil Pollution Abatement Equipment: Location – Cape Town

|  | Description | Quantity |
| --- | --- | --- |
| 1 | Storage Tank-tainers 20 m3 | 5 |
| 2 | Storage Tanks 35m3 | 2 |
| 3 | Hoses |  |
| 4 | a. Rigid camlock b. Rigid camlock c. Flanged d. Flanged e. Flanged f. Flanged g. Flanged h. Kanaflex | 3" 89 x 1210m 4" 21 x 63m 6" 41 x 290m 8" 29 x 300m 10" 16x 158m 12" 12 x 140 m 6" layflat 11 x 300m 2" 19 x 190m Kanaflex 3" 33 x 420m Kanaflex 4" 33 x 635m Kanaflex 6" 44 x 360m |
| 5 | Diesel / Hydraulic Powerpacks | 4 x 120 kw 10 x 25 kw 2x 46 kw |
| 6 | Hydraulic submersible pumps | 3" 4 4" 3 6" 3 |
| 7 | Hydraulic Oil Transfer pumps | 3" 8 4" 3 6" 6 |
| 8 | Hydraulic Hoses for Hydraulic Pumps | 37 x 860 m |
| 9 | Yokohama Fenders with cradles | 6 x 7m x 3m 2 x 2m x 1,5m |
| 10 | Yokohama Fender Pressure test tank | 1 |
| 11 | Tripods | 15 |
| 12 | STS Cages | 3 |
| 13 | Includes reducers & Blanks | 16"; 12"; and 10" |
| 14 | STS Toolboxes | 4 |
| 15 | Reducers |  |
|  | 6"-4" | x 8 |
|  | 6"- 8" | x 13 |
|  | 6" — 10" | x 2 |
|  | 8" - 12" | x 1 |
|  | 8" - 4" | x 1 |
|  | 8" - 8" | x 2 |
|  | 8" — 10" | x 6 |
|  | 10"-10" | x 9 |
|  | 10"- 12" | x 9 |
|  | 10" - 16" | x 1 |
|  | 12" - 12" | x 1 |
|  | 12" - 8" | x 1 |
| 16 | Blank & Pulling Flanges | 4" x 1 6" x 15 8" x 14 10" x 25 12" x 15 14" x 1 16" x 2 18" x 1 |
| 17 | Ropes | 96 mm x 5 |
| 18 | Full coils | 88 mm x 7 64 mm x 2 56 mm x 4 44 mm x 1 |
| 19 | Part Coils with eyes 36 - 96 mm | x 103 |
| 20 | Seaslug Floating Storage Dracone Capacity 94 600 litres | 1 |
| 21 | Floatation Bags Lifting capacity 760 kgs | 36 |
| 22 | Bladder Tanks Capacity 22000 litres | 8 |
| 23 | Emergency Response Pollution Containers | 3 |
| 24 | Oil absorbent Peat Bags | 65 |
| 25 | Oil absorbent Booms | 8 |
| 26 | Chemical Safety Suits | 50 |
| 27 | Aluminium Skimmer unit, manual | 1 |
| 28 | Storage Tanks, Plastic Cap 1 m3 | 4 |
| 29 | Air Drills | 2 |
| 30 | HP Water Washing machines with Foam attachments Capacity | 5 |
| 31 | Chemical Drum Pumps with Hoses | 4 |
| 32 | EX Safety Lamps (Air driven) | 4 |
| 33 | Spray Booms | 2 |
| 34 | Breaker Boards | 20 |
| 35 | Rapid Response Trailer | 1 |
| 36 | SMIT Towing Brackets | 2 |
| 37 | Sludge storage tank | 2,5m3 x 1 |